WATER SPORTS CERTIFIED QUALITY PROTOCOL

# Version 2

January 2018

Version 2 - January 2018 	Page 1 of 121

The owner of this document is TÜV AUSTRIA HELLAS SINGLE-MEMBER LLC

Technical Audits - Inspections 
Certifications - Training

429 Mesogeion Avenue, 15343, Agia Paraskevi

TÜV AUSTRIA HELLAS SINGLE-MEMBER LLC retains all rights. Any reproduction of all or part of the document in any way, print or electronic, is prohibited without the written consent of the owner of the Protocol. Any unauthorised use is prohibited and subject to legal action.

The Water Sports Certified Quality Protocol is available online on the website

http://www.tuvaustriahellas.gr/ or via email at

certification@tuvaustriahellas.gr

Acknowledgements

TÜV AUSTRIA HELLAS SINGLE-MEMBER LLC wishes to thank all participants who have responded earnestly to the invitation for the development of this Protocol. Their information, views, accumulated knowledge and experience was a great support to TÜV AUSTRIA HELLAS. We value the time they devoted to responding positively to the endeavour to create a Private Protocol that will lend added value to our country's tourism product.

The aim of this private initiative is to promote the sense of safety of users of water sports and to improve the quality of service provided by the Licensed Water Sports Centres.

TÜV AUSTRIA HELLAS recognising their contribution, would like to thank:

| Panagiotis Paschalakis | President of the Non-Profit Organisation, Safe Water Sports |
| --- | --- |
| NOSIS | Business Consultants |
| Directorate of Port Police | Department of Policing and Order Maintenance Measures |
| Paraskevas Panagiotis | Rear Admiral of the Coast Guard |

Members of the Technical Committee for the drafting of the Water Sports Certified Quality Protocol:

| Charalambos Aggeloudis | Director of Management Systems and Product Certification |
| --- | --- |
| Maria Agapitou | Deputy Director of Management Systems and Product Certification |
| Evangelia Giannakopoulou | Quality Management Systems Manager of the Management Systems and Product Certification Division |
| Konstantinos Mavropoulos | Development Manager of the Management Systems and Product Certification Division |

WATER SPORTS CERTIFIED QUALITY

PROTOCOL

# Version 2

January 2018

### Contents

- | Purpose	9

- | Scope	 9

- |Definitions	9

  - Licensed Water Sports Centre	9

  - Operator	 10

  - Maritime Recreational Means / Water Sports 	10

  - Certification Body 	13

  - Audit	 13

  - Interested parties	 13

  - Parties involved 	13

  - Seasonal operation of Licensed Water Sports Centres (season) 	13

- | Audit Categories	 14

  - Initial Certification Audit	14

  - Renewal Audit	 14

  - Repeat Audit	 15

  - Extraordinary Audit	 15

  - Unannounced Audit	 16

- |Certification Procedure	 16

  - Planning of Audit:	16

  - Conclusion of Collaboration Agreement and Audit Scheduling:	17

  - Compliant Companies Classification System	 17

  - Criteria evaluation system	 18

  - Reporting of Results	18

  - Managing Deviations / Non-Conformities	 19

  - Calculation of Audit Duration	 19

  - Certification Cycle:	20

  - Shape, Form and Structure of Certificate:	20

- Mandatory (Fundamental) audit criteria for certification 	21

  - Organisation and operation	 22

  - Staff and Responsibilities	 22

  - Maritime Recreational Means Service Performance	 24

  - Support Equipment for Maritime Recreational Means Services	 28

  - Support Facilities for Maritime recreational Means 	29

  - Improving Services and Performance 	30

- | Optional (Basic) Requirements for Classification of Certification 	32

  - Organisation and operation	 31

  - Staff and Responsibilities 	31

  - Performance of Maritime Recreational Means Services	 32

  - Support Equipment for Maritime Recreational Means Services	 34

  - Support Facilities for Maritime recreational Means 	35

  - Improving Services and Performance 	35

- |Management of Complaints / Appeals / Grievances	36

  - Complaints	 36

  - Appeals	 36

  - Handling Complaints Related to Audit Requirements	37

  - Third Party Grievance	37

  - Documentation and Monitoring	37

- |Revocation of Certification	38

- |Ownership and Management of the Certification Mark	39

  - Expiry of Right to Use	40

- | Scheme Safeguarding	 40

  - Reliability	 40

  - Independence / Impartiality / Integrity 	41

  - Confidentiality 	41

  - Integrity Committee 	41

  - Internal Audit	 42

- Auditor Requirements	42

  - Auditor Training	42

  - Methodology for the Approval of Inspectors	43

    - Initiating Collaboration	 43

    - Monitoring Collaboration	 43

- | Motorised Maritime Recreational Means	 45

  - 1. Water skiing	 46

13.1. 2. Parasailing or Parachuting	 46

13.1. 3. Water Towable Tubes	 48

13.1. 4. Inflatable Maritime Recreational Means (e.g. water towable ring, etc.)	48

13.1. 5. Flyfish - Flying Banana	 49

13.2A. 1. Personal water crafts (SeaBeetle, SeaBike, Jet-Surf, Jet Ski etc.) 	50

13.2A. 2. Personal water crafts (SeaBeetle, SeaBike, Jet-Surf,

Jet Ski, etc.) 	50

13.2A. 3.  Flyboard- Jetovator- X Jetpack	 52

13.2A. 4.  Flyboard- Jetovator- X Jetpack	 52

13.2A. 5. Hover Craft	 53

13.2A. 6. Motor boats 	55

13.2A. 7.Oxoon Jet / Oxoon Jet / Oxoon New Generation /Hydroxoon F1 Maritime Recreational Means	 55

13.2A. 8.  Hawaii, Hawaii GT, Adventurer and Adventurer GT	 57

13.2B. 1. Aqua Scooter / Sea - Doo Sea Scooter	58

13.2B. 2. Mini Cruiser	 58

13.2B. 3. Bbq Donut	 59

13.2B. 4. Funboat - Chaiselounger 	60

13.2B. 5. PowerSurf XS	 61

13.2B. 6. Waterbuggy	 61

13.2B. 7. Sea Spi single and Sea Spi twin carries	 62

13.2B. 8. Catamaran with Electric Propulsion	 63

- | Non-Motorised Maritime Recreational Means	 64

  - 2. Kite Surfing 	65

14.1. 3. Catamaran	 67

  - 4. Sailing boats	 67

  - 1. Pedal Boats	 68

14.2. 2. Seabuggy	 69

14.2. 3. Aqua Paddle Boats	 69

  - 4. Acrylicraft	 70

  - 1. Canoe - Kayak	 71

  - 2. Stand-Up Paddle board (SUP) 	 71

  - 1. Waterblob	 72

14.4. 2. Inflatable Water Park	 73

14.4. 3. Water Polo Goal Posts, Basketball Hoops, Volleyball Nets	74

- Annex 2	 75

15.1 Audit Forms	 75

  - Audit Report	 75

  - Audit Checklist	 80

- Annex 3 - Application for inclusion under Quality Label	 115

- Annex 4 - Certificate	 117

## |Purpose

Primary purpose: The primary purpose of the Protocol is to define and evaluate the responsiveness of Licensed Water Sports Centres regarding the Fundamental Requirements concerning their operating mechanism, and in particular the quality of the services they provide, as well as their customer support and the level of their available infrastructure. For this purpose which contributes to the safe provision of services, by the Licensed Water Sports Centres to the users of Maritime Recreational Means, representative requirements have been taken into account, pursuant also to the regulatory framework for the operation of Licensed Water Sports Centres.

Secondary Purpose: The secondary purpose of the Protocol is to define and evaluate Basic Requirements for the continuous improvement and upgrading of company services in all areas of the primary purpose of the Protocol. Specifically, in the documentation for quality support and the management of data and information regarding the operation of Licensed Water Sports Centres, as well as of the level of support to customers, and the organisation of the companies themselves.

## |Scope

The scope of the Protocol applies solely to Licensed Water Sports Centres for the activities they carry out and the Maritime Recreational Means they offer, based on their operating license, as well as the related services they provide associated with these.

## |Definitions

3.1.Licensed Water Sports Centres:

Licensed Water Sports Centres are those whose activity is the rental of maritime recreational means.

3.2Operator

Operator, means the legal representative or owner of the Licensed Water Sports Centres.

3.3.Maritime Recreational Means / Water Sports

Maritime Recreational Means are those defined in General Port Regulation No. 20 (Government Gazette B444/26-04-1999) and its amendments, namely:

  - Water skiing

A speedboat, using a suitable rope, tows an individual who moves across the surface of the water with the help of special waterskis

  - Parasailing or Parachuting

The elevated flight of an individual by means of a special parachute towed by a suitable speedboat.

  - Towable Inflatable Maritime Recreational Means

A speedboat tows an inflatable recreational means, with a plastic floor and without a cover (on any part of its surface), which has handles for passengers to hold on to, does not fly and has been designed for use by an individual, or individuals, seated or lying face down. The towed inflatable maritime recreational means carries as many passengers as there are pairs of handles for them to hold on to.

  - Sea Sled (Water Towable Tube)

A speedboat tows an inflatable recreational means consisting of a single central cylindrical float and two smaller floats permanently attached on the left and right of it. The sea sled carries as many passengers as there are pairs of handles for them to hold on to.

  - Water Towable Ring

A speedboat tows an inflatable circular recreational means with a plastic floor, which has handles for passengers to hold on to. The water towable ring carries as many passengers as there are pairs of handles for them to hold on to.

  - Personal water crafts (SeaBeetle, SeaBike, Jet-Surf, Jet Ski etc.)

A floating petrol-powered means which can carry up to three (3) persons depending on its construction type.

  - Windsurf

A long, watertight float on which a small mast with a sail is placed, which is powered by the wind and the appropriate handling of the sail by the passenger.

  - AquaScooter

A small petrol-powered motor, which has the ability to float and tow the swimmer in a face down or supine position.

  - Seabuggy

A small inflatable device which can carry a single individual (in a face down or supine position), who will drive the device forward in the sea with the strength of their leg muscles.

  - Mini Cruiser

A small motorised inflatable device on which a single person lies along its length in a face down position.

  - Canoes and Kayaks

are vessels with a hull width of less than 1.1 metres, pointed at both ends, which move with a special unhinged paddle having no permanent position for fixing an engine to for their propulsion, and no more than one paddle per passenger, nor sails

  - Hovercraft

A motorised maritime means which moves on a layer of air both on the shore (smooth surface) and in the sea. A propeller is used as a means of creating the air layer and its propulsion, located in the aft part of the boat.

  - Acrylicraft

A small boat made of transparent acrylic material without mechanical means of propulsion. Within it, transversely, there are plastic boxes used for seating, which are watertight and serve as a means of lifting in the event that the boat capsizes. Along its gunwale, on the left and right, there is an aluminium reinforcement with a suitable socket for fixing the paddles and watertight boxes. There are two aluminium paddles available in each craft.

  - Pedal boats (AquaBike, Explorer, etc.)

A floating recreational means consisting of long, interconnected, watertight floats. On the formed raft there are up to four (4) seats or a common bicycle, depending on its construction. The movement of the pedal boat is achieved by the use of the muscular force in the legs of the passenger(s)

  - Catamaran

A floating recreational means consisting of two (2) elongated, interconnected, watertight floats. On the formed raft there is a plastic covering on which the passenger is seated and at a suitable point a mast with a corresponding sail is fitted. This floating means is powered by the wind and the suitable handling by the passengers

  - Motor Boat

A boat with a motor (engine) which does not fall within the meaning of a speedboat (fast) and may be rented without the lessor being on board

  - Sailing Boat

A boat without a deck (cover), which is driven by the wind and for this reason has a main mast with sails and a keel suitable for its stability

  - Aqua Paddle Boat

  - BBQ Donut

  - Personal water craft for hydropower elevation - Flyboard

An accessory component, for a personal water craft, in the form of a special attachment - footrest system with nozzles at the feet and hands. In particular, the aforementioned accessory is connected to a jet ski-type craft by means of a pipe through which water is forced under pressure and the thrust of the water is used to lift the accessory and the user into the air, said user has the ability to redirect the thrust of the water.

  - Flyfish 6 Pro, Flyfish 2 Pro and Flyfish small

  - Flying banana

  - Fun boat

  - Chaise lounger

  - HAWAII, HAWAII GT, ADVENTURER, ADVENTURER GT, HYDROXOON F1, OXOON JUNIOR JET, OXOON NEW GENERATION, POWER SURF XS, SEA SPI SINGLE, SEA SPI TWEEN CARRIES, SEA - DOO SEA SCOOTER Maritime Recreational Means

  - Personal water craft for hydropower elevation - JETOVATOR

  - Kitesurfing

  - Stand-Up Paddle board (SUP)

  - Water buggy

  - WATERBLOB

watertight float used by two (2) users at the same time. Users board a floating platform or craft and the first user (blobber)

takes position on the float at its farthest point. The second user (jumper) jumps onto the float, causing the first to be launched into the air and to land in the sea.

  - Inflatable Water Park

A system of maritime recreational means, which includes inflatable water games such as trampolines, seesaws, slides, a climbing mountain and more.

  - Catamaran with Electric Propulsion

  - Water polo goal posts, basketball hoops, and volleyball nets which are placed in the sea and supported by styrofoam or other similar material.

  - Any other Maritime Recreational Means not listed in the General Regulation Port No.20, yet licensed as a novelty and listed on the rental license.

3.4.Certification Body

The Certifying Body of this Protocol is TÜV AUSTRIA HELLAS, hereinafter referred to as TAH, as an independent organisation providing integrated auditing services.

3.5.Audit

An audit is a process of examining a service or its design and of determining its compliance with specific requirements or, on the basis of professional judgement, with general requirements.

3.6Interested parties

Interested parties are the users of maritime recreational means, the institutional factors, the local communities, etc.

3.7.Parties involved

The parties involved for this Protocol are the Certification Body, TÜV AUSTRIA HELLAS, the Non-Profit Organisation, Safe Water Sports, and the Licensed Water Sports Centres.

3.8Seasonal operation of Licensed Water Sports Centres (season)

The seasonal operation of Licensed Water Sports Centres is defined as the period from 1 March of each year to 31 October of each year.

## |Audit Categories

The classification of audits is based on their purpose. All audits are always carried out within the Licensed Water Sports Centre's operating period.

The company must send to the certification body the relevant

"Statement of Satisfaction of Optional (Basic) Requirements" form of the 'Water Sports Certified Quality' protocol at least 10 days prior to the audit (except in the case of an unannounced audit).

Specifically, the audit categories are as follows:

4.1.Initial Certification Audit

The Initial Certification Audit is the audit of initial inclusion of the company under the 'Water Sports Certified Quality' Quality Label. It is conducted at a time and date that is agreed between the company and TAH. During this audit the company shall be audited as a whole, in relation to its documentation, the relevant procedures and in accordance to the requirements criteria of this Protocol, that is to say, the entire set of Fundamental Requirements and also those of the Basic Requirements to which the Licensed Water Sports Centre complies and on the basis of which it will receive the relevant classification of its certification (see par.5.3).The basic requirements for which the Licensed Water Sports Centre will be assessed by the Certification Body's inspection team are highlighted in the relevant "Statement of Satisfaction of Optional (Basic) Requirements" form submitted by the Licensed Water Sports Centre to the Certification Body, before each audit.

4.2.Renewal Audit

The Renewal Audit is the audit that is conducted after the initial audit. The renewal audit takes place one year after the initial certification audit, at the latest. The renewal audit includes a complete and thorough audit of a company, resulting in the issue of a new certificate.

During the Renewal Audit, the auditor assesses all Fundamental Requirements related to the activity of the Licensed Water Sports Centre and the Basic Requirements, as declared by the Licensed Water Sports Centre in the relevant checklist prior to conducting the audit. Particular attention is paid to the deviations and non-conformities identified during the Initial Certification Audit or the previous, most recent Renewal Audit - whichever has been conducted closest to this Renewal Audit - as well as the effectiveness and implementation of corrective actions and preventive measures defined in the corrective actions plan of the company.

Note: corrective action plans from previous audits shall always be assessed by the auditor during the next audit, even if the previous audit has been performed more than one year prior.

The date of the Renewal Certification Audit is calculated from the date of initial certification audit rather than the date of issue of the corresponding certificate.

4.3Repeat Audit

A Repeat Audit is the audit conducted in the case in which, during the Audit for Initial Certification or the Renewal of Certification validity, the on-site re-evaluation of the Licensed Water Sports Centre is deemed necessary by the auditor. This may occur in the case of Non-Conformity with regard to a claim which cannot be removed by sending supporting evidence and/or in the case in which the Audit for Initial Certification or the Renewal of Certification validity is interrupted.

During the Repeat Audit, the auditor focuses on the implementation of the actions taken to correct the non-conformities, with regard to the requirements, arising from the Initial Certification Audit or the Renewal of Certification validity. The Repeat Audit is carried out within one month of the date of the last audit. If there are no reasons of force majeure, the auditor who conducted the last certification audit should also conduct the Repeat Audit.

If the Repeat Audit is not performed within one month of the date of the Initial Certification Audit, at the fault of the Licensed Water Sports Centre, then an Initial Certification Audit is required once again. In the event that, during the Repeat Audit, it appears that there is no evidence of rectification of the Non-Conformities, an Initial Certification Audit is required once again within a reasonable time.

The elimination of non-conformities under the fundamental and/or basic requirements is achieved either by sending corresponding evidence or by an on-site visit, if their elimination can not be verified by sending evidence.

4.4.Extraordinary Audit

The Extraordinary Audit is the audit conducted at an unscheduled interval for a number of reasons such as, the extension of the scope, the investigation of a complaint or grievance, and so on. TAH is responsible for setting the relevant requirements for the audit and its duration, depending on the reason for the Extraordinary Audit. Therefore, in cases of a complaint or grievance, an audit over the entire company is conducted regarding its documentation, the relevant procedures and always in accordance with the requirement criteria of this Protocol.

In cases of the extension of the scope, the audit focuses on those activities concerning and affecting the new scope, and if the company fulfils the conditions for extending the scope, the certificate is reissued with the new scope. The new certificate has the same expiry date as the previous valid certificate.

If during the Extraordinary Audit a Non-Conformity is found in regard to a fundamental requirement or a basic requirement which affects the Assessment/Classification Rating of the Licensed Water Sports Centre, then the certificate in force is revoked and a new initial Certification audit is required in order to reintegrate the company under the ‘Water Sports Certified Quality' Quality Label.

4.5Unannounced Audit

The Unannounced Audit is the audit carried out by TAH, without prior notice, on the premises of the Licensed Water Sports Centre. The Unannounced Audit is carried out to verify that the Licensed Water Sports Centre has and can demonstrate the required and necessary degree of preparedness and responsiveness with regard to its compliance to the requirements of this Protocol, even at a time beyond that specified by an expected scheduled audit.

An unannounced audit is carried out in companies for which the risk assessment, which is carried out by the certification body after the end of the previous inspection/certification cycle, indicates the need to reassess the Licensed Water Sports Centre's compliance with the requirements of the Scheme, without prior notice.

Criteria for conducting an Unannounced Audit may also be carried out in cases where TAH receives grievances about the Certified Licensed Water Sports Centre, or in the case of repeated negative comments about the services of the Certified Licensed Water Sports Centre.

## |Certification Procedure

The certification procedure defines the stages and steps followed on behalf of the Licensed Water Sports Centre, from receiving the application for inclusion under the "Water Sports Certified Quality" Quality Label to the award of the certificate. Specifically, the steps are as follows:

5.1.Planning of Audit:

The planning of the audit concerns the review and evaluation of information relative to the Licensed Water Sports Centre interested to be included, or to extend its scope or to maintain its certificate with regard to the 'Water Sports Certified Quality' Quality Label, in order to determine the characteristics and requirements of the audit.

For the inclusion, or expansion, or for any change in the activities of a Licensed Water Sports Centre in relation to those services and offered Maritime Recreational Means initially declared and/or certified under the 'Water Sports Certified Quality' Quality Label, the Licensed Water Sports Centre completes the Assessment Application. In it, it records the details relating to its identity and its activities. In addition, the Licensed Water Sports Centre completes the relevant "Statement of Satisfaction of Optional (Basic) Requirements" form regarding the Protocol requirements, relative to its activities, associated with the classification of its certification. These details are reviewed by TAH in order to determine the characteristics of the inspection (time duration, inspection team, cost etc.).

The certification concerns all Maritime Recreational Means listed on the Licensed Water Sports Centre's license without giving the option to exclude any of these.

The Licensed Water Sports Centre is obliged to immediately inform TAH about any change to the Maritime Recreational Means rental license within a reasonable time before the final scheduling of the audit.

5.2.Conclusion of Collaboration Agreement and Audit Scheduling:

Upon the conclusion of the Collaboration Agreement between the Licensed Water Sports Centre and TAH regarding the 'Water Sports Certified Quality' Quality Label, Audit Scheduling follows, defining the precise details and conditions for conducting the audit, with regard to:

- Audit team, which has the required competence for the required scope - activities

- Audit duration, which shall be sufficient to audit all the requirements of the Protocol

- Determination of audit timing, in agreement with the Licensed Water Sports Centre

5.3.Compliant Companies Classification System

The Licensed Water Sports Centre, which complies with at least 100% of the Fundamental Requirements of the Protocol (related to the subject of its services), may receive the "Water Sports Certified Quality" Certification with a classification characterisation: Bronze Water Sports' Centre (Bronze Water Sports'

Centre).

However, the Licensed Water Sports Centres which, in addition to the Fundamental Requirements, also comply with a specific number (percentage) of the Basic Requirements of Paragraph 7 of the Protocol, receive the “Water Sports Certified Quality” Certification with the following higher Certification Classification Levels, based on the percentage of Basic Requirements which they fulfil and which fall within the scope of their certification:

Silver Water Sports' Centre (Silver Water Sports' Centre)

“Water Sports Certified Quality” Certification Classification Level:

Up to and including 40% compliance with Basic Requirements.

Golden Water Sports' Centre (Golden Water Sports' Centre)

“Water Sports Certified Quality” Certification Classification Level:

From 40%, up to and including 75% compliance with Basic Requirements

Platinum Water Sports' Centre (Platinum Water Sports' Centre)

“Water Sports Certified Quality” Certification Classification Level:

Over 75% compliance with Basic Requirements

Compliance with each specific Basic Requirement presumes the complete and documented response to it by the Licensed Water Sports Centre.

The Assessment/Classification of the company is directly related to the period of validity of the corresponding certificate, subject to the continued adherence to the relevant requirements throughout the duration of its validity. As a result, the Assessment/Classification Rating of a Licensed Water Sports Centre, may be modified (upgraded or downgraded or even revoked) depending on the results of subsequent audits, of any kind of those mentioned above in this document.

5.4.Criteria evaluation system

Non-Compliance: Non-Compliance concerns the non-adherence (total or partial) to Fundamental Requirements related to the Primary Purpose of the Protocol and/or the Basic Requirements related to the Unit's Assessment/Classification Rating.

Deviation: A deviation is characterised by a tendency that could potentially lead to non-adherence (in whole or in part) to the Fundamental Requirements, which may therefore affect the Primary Purpose of the Protocol and/or the Basic Requirements which may or may not affect the Assessment/Classification Rating of the unit.

It should be noted that a Licensed Water Sports Centre cannot be assessed with respect to those criteria of the Protocol which documented do not relate to the scope of its activity.

5.5Reporting of results:

A complete report of the results is prepared after each audit.
The report is divided into different sections:

- Information regarding the identity of the Licensed Water Sports Centre (Full Name, Address / Installation Site / Tax ID Number, etc.)

- Precise description of the activities and of the maritime recreational means

- Audit result

- Summary in tabular form for all chapters.

- Recapitulation of all chapters and observations relative to the monitoring of corrective actions carried out since the previous audit.

- Observations relative to Protocol requirements

- Separate list (including explanations) of all requirements that have been evaluated

- Table of all deviations, non-conformities

- Approval and release of the audit report by the Management Systems and Product Certification Division of TAH, for the issue of the certificate.

5.6Managing Deviations / Non-Conformities:

All Deviations / Non-Conformities to the Fundamental or Basic Requirements are communicated at the time they are identified and recorded in the audit report. The audited Licensed Water Sports Centre receives the audit report and determines the corrective actions that it will subsequently implement. The auditor verifies the corrective actions either by reviewing the evidence or by performing a repeat audit.

The audit report is signed by the auditor, while the owner or legal representative of the Licensed Water Sports Centre co-signs and stamps, in a specified field, for the acceptance of the recorded audit findings.

A Licensed Water Sports Centre may decide not to take a corrective action in respect to a Non-Conformity or Deviation on a Basic Requirement, provided that the Assessment/Classification Rating of the unit, as stated in the Certificate of Conformity or Initial Audit, is not affected.

5.7.Calculation of Audit Duration

The minimum duration of the inspection is one man-day (MD) of 8 hours and includes the time for the on-site audit and the writing of the audit report

The duration of the inspection shall be calculated on a case-by-case basis taking into account the following factors:

- Total number of staff involved

- the total number of water sports and recreational equipment

- the total area occupied by the Licensed Water Sports Centre

5.8Certification Cycle:

The duration of the certification commences on the validity commencement date and with the issue of the certificate, and has a duration of one calendar year (12 months).

The validity commencement of the certification is defined as the confirmation time of the full compliance of the Licensed Water Sports Centre with the requirements of the Protocol, following the on-site Audit (Initial Certification or Renewal), carried out by the Audit Team of the certification body, TAH, and the implementation of any corrective actions by the Licensed Water Sports Centre for any recorded Non-Conformities.

A required condition for the validity of the Certificate of a Licensed Water Sports Centre is the valid operating license of the company (Licensed Water Sports Centre).For any period of time within the certification cycle in which the Licensed Water Sports Centre's operating license is not active, in accordance with the provisions that the competent State Authority adopts for the granting and the duration of validity of a rental license for maritime recreational means, the provisions of the certification and the responsibilities arising therefrom for the certification body, TAH, shall immediately cease to be valid, as well as from the Certification Agreement of the Protocol between the two parties. They shall be reinstated as valid once again at the date of activation (renewal) of the license of the Licensed Water Sports Centre and if it is within the validity period of the Certificate. To this end, the Licensed Water Sports Centre is obliged to inform the certification body, TAH, in writing of the expiry date as well as of the restart time of its operating license (Rental of Maritime Recreational Means).In order for the company to re-use in any way the 'Water Sports Certified Quality' Certification after the re-activation (renewal) of the validity of its operating license, it must, in addition to the above, provide in advance to the certification body, TAH, the renewal of its renewed operating license or other equivalent document issued by the Competent Authority to this end.

For the renewal of the company’s certification, the relative audit is carried out in a timely manner before the expiry of the previous certification, so that any corrective action can be implemented regarding non-conformities of the renewal audit, within that time.

The new certificate will be issued in succession to the previous certificate, with the same registration number.

5.9Shape, Form and Structure of Certificate:

TAH issues a Certificate of Compliance for the requirements of this Protocol.The certificate mentions:

- The name of the Protocol and the audit criteria

- The name and address and / or installation site of the Licensed Water Sports Centre.

- The scope - the maritime recreational means for which certification is valid

- The entry into force and expiry date of the validity of the certification. In the case of renewal of the certification, all previous certification periods will be displayed.

- The unique registration number of the certificate

- The name, brand name and address of TAH

- Discrete holographic-type marking with the TAH brand name to prevent the unlawful reproduction of the certificate

- A declaration stating that the certificate belongs to TAH and that it is forbidden to reproduce it in any way

The issue of the Certificate and the Quality Label may also be announced in the Safe Water Sports electronic application.

The certificate shall have the form as shown in Annex 4 of this Protocol.

## |Mandatory (Fundamental) audit criteria for certification

The following criteria pertain to the mandatory criteria (requirements) that must be satisfied by each Licensed Water Sports Centre for its certification according to the "Water Sports Certified Quality" Protocol.

These criteria are audited during the on-site audit of the Licensed Water Sports Centre in order to assess compliance for the certification and receipt of the Quality Label, "Water Sports Certified Quality" - Certified Centre.

The criteria fall within the following 6 categories.

- Organising the company's operational framework

- Staff & Responsibilities

- Performance & Provision of Water Sports

- Support Equipment for Maritime Recreational Means Services

- Company facilities

- Improvement of Services

6.1.Organisation and operation

|  | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.1.1 | Fundamental | The Licensed Water Sports Centre has a copy of the valid rental license available at the Licensed Water Sports Centre's post where services are provided, and in a prominent place. |
| 6.1.2 | Fundamental | The Licensed Water Sports Centre has insurance policies for each craft (and the "rescue" boat) and for the Maritime Recreational Means that fall under the specific legislative requirement, which are legally in its possession and which are used for the services it provides. The insurance policy covers third party liability insurance for death, personal injuries and material damage, in force. |
| 6.1.3 | Fundamental | The Licensed Water Sports Centre has licenses to sail in place for each craft (and the "rescue" boat). |
| 6.1.4 | Fundamental | The Licensed Water Sports Centre records by type (motorised and not) all available Maritime Recreational Means for rent (e.g. speedboat, jet ski, water ring, kitesurf, etc.), and verifies the correct recording of names and Small Boat Registration Books (ΒΕΜΣ) numbers (for any Maritime Recreational Means subject to such a requirement). |

6.2.Staff and Responsibilities

| S/N | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.2.1. | Fundamental | The Licensed Water Sports Centre assigns the implementation of the work to staff with the necessary (special) training and qualifications as defined by the legal and regulatory framework, and also by the company policy. |

| S/N | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.2.2 | Fundamental | If the rental license includes as maritime recreational means for rent, water skiing and/or parasailing and/or the sea sled and/or the water towable ring and/or any other which is similar to these and is towed by a speedboat, the responsible lessor possesses (a) a water skiing instructor's license of any category under the General Secretariat for Sports (GSS) and a speedboat driving license for at least two years or (b) is a graduate of the Faculty of Physical Education and Sports Science with a specialisation in water skiing and has, for at least one year, a speedboat driving license or (c) already holds a valid water skiing instructor's license, whose validity has been certified under the provisions of the General Port Regulations and its holder has previously practiced, under the authorisation of the Port Authority, the profession of lessor or has provided actual and active services to a lessor for maritime recreational means towed by a speedboat. |
| 6.2.3 | Fundamental | Subsequent to the previous audit point, provided that the Licensed Water Sports Centre has available assistant(s) to the above responsible lessor, this assistant(s) is occupied with the performance of water skiing and/or parasailing and/or the sea sled and/or the water towable ring and/or any other which is similar to that towed by a speedboat and for this purpose has been audited and either: (a) holds a water skiing instructor's license of any category under the General Secretariat for Sports (GSS) and a speedboat driving license for at least two years or (b) is a graduate of the Faculty of Physical Education and Sports Science with a specialisation in water skiing and has, for at least one year, a speedboat driving license or (c) already holds a valid water skiing instructor's license, whose validity has been certified under the provisions of the General Port Regulations and its holder has previously practiced, under the authorisation of the Port Authority, the profession of lessor or has provided actual and active services to a lessor for maritime recreational means towed by a speedboat. |

6.3.Performance of Maritime Recreational Means Services

| S/N | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.3.1. | Fundamental | The Licensed Water Sports Centre rents out only maritime recreational means, by type and number, which are included in the rental license. |
| 6.3.2. | Fundamental | While the Maritime Recreational Means are being towed by a speedboat, there is a second person on board besides the qualified driver (according to par. 6.2.9) for the continuous monitoring (supervision) of the practitioner, who has received the necessary briefing. |
| 6.3.3. | Fundamental | The Licensed Water Sports Centre documents the rental of a maximum of three personal water crafts at any given time, which it has in its possession and for which the legislative documents are available. |
| 6.3.4. | Fundamental | The company evaluates and documents the competence of users of motorised equipment, e.g. jet ski, for which knowledge of how to handle them is required before their use. |
| 6.3.5. | Fundamental | The Licensed Water Sports Centre has captured and monitors all the potential routes of all types of motorised maritime recreational means, for which (routes) it notifies users (regarding these and for the other Maritime Recreational Means). |
| 6.3.6. | Fundamental | For the description of each potential route of the floating Maritime Recreational Means, parameters which may affect their motion, have been taken into account and are marked or recorded on a relevant sketch and are communicated to the users. |
| 6.3.7. | Fundamental | The lessor does not himself engage in a simultaneous activity (e.g. the use of seats or sun-beds or shading equipment, etc.), which may affect the safe provision of the services of the Licensed Water Sports Centre. |
| 6.3.8 | Fundamental | The lessor does not employ and does not allow the employment of his staff, whose duties are to support the Maritime Recreational Means services, in other commercial activities (e.g. rental of beach umbrellas, etc.). |

| S/N | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.3.9 | Fundamental | The activities within the area of business activity of the Licensed Water Sports Centre (in and out of the sea), which may affect the safe provision of its services, have been taken into account and assessed adequately by the same (e.g. operation of another Licensed Water Sports Centre, anchorage, etc.). |
| 6.3.10 | Fundamental | The Licensed Water Sports Centre possesses a motor boat with features which allow it to be used as a rescue boat. |
| 6.3.11 | Fundamental | The Licensed Water Sports Centre does not rent out the rescue boat and/or the use of the speedboat, which serves as a rescue boat, for any other purpose other than the provision of rescue services. |
| 6.3.12 | Fundamental | The lessor or the employee documents their knowledge of Basic Life Support (primary assessment, CPR, rescue breaths). |
| 6.3.13 | Fundamental | The Licensed Water Sports Centre is supported by a lifeguard whose competence is documented. |
| 6.3.14 | Fundamental | The lessor, from the point where he is located can in fact reasonably observe as much as possible the rented recreational means, to provide assistance in case of danger. |
| 6.3.15 | Fundamental | The Licensed Water Sports Centre does not allow renting to persons under the influence of alcohol or toxic substances. |
| 6.3.16 | Fundamental | The Licensed Water Sports Centre has posted a copy of the price list of the services at a suitable point of the post (e.g. on the shore), in the Greek and English language. |
| 6.3.17 | Fundamental | The Licensed Water Sports Centre has the capability to store all the Maritime Recreational Means after the end of the working day in order to ensure that they will not be used. |
| 6.3.18 | Fundamental | The boat, which may be used at any time as a rescue boat, is in a point and in a condition such that it can immediately provide rescue services to users of Maritime Recreational Means (e.g. maintenance, sufficient amount of fuel, etc.). |

| S/N | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.3.19 | Fundamental | The Licensed Water Sports Centre has a clearly identified waterway as a means of transit of the maritime recreational means, during their travel from and to the launching point, where this is foreseen and required. |
| 6.3.20 | Fundamental | The Licensed Water Sports Centre has installed a special buoy (mooring) to be used by the lessor as a means of launching the motorised recreational means, provided this is stated on the license. |
| 6.3.21 | Fundamental | The Licensed Water Sports Centre has anchored its own private or specially chartered boat, to be used by the lessor as a means of launching the motorised recreational means, provided this is stated on the license. |
| 6.3.22 | Fundamental | Subsequent to the previous audit point, the boat has raised the anchorage signals prescribed by the International Regulations for the Prevention of Marine Collisions. |
| 6.3.23 | Fundamental | In the area of activity of the Licensed Water Sports Centre, the area of travel of the rented crafts and the bathers is distinguishable. |
| 6.3.24 | Fundamental | All Maritime Recreational Means are regularly checked and maintained by the Licensed Water Sports Centre without altering the features specified by the manufacturer. |
| 6.3.25 | Fundamental | All life jackets made available by the Licensed Water Sports Centre have a bright colour, except those used for water skiing. |
| 6.3.26 | Fundamental | The Licensed Water Sports Centre provides life jackets of appropriate size and shape to users of Maritime Recreational Means and checks their proper use by practitioners prior to commencement of the activity. |
| 6.3.27 | Fundamental | All passengers of Maritime Recreational Means wear life jackets throughout the duration of their use. The lessor and his assistant may be exempt from this obligation. |
| 6.3.28 | Fundamental | For users that are minors, the company has a consent form to participate in the activity. The form is signed by the custodian or the parent or the supervisor of the minor, or it simply accompanies the minor. In this the minor's ability to swim must be confirmed. |

| S/N | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.3.29 | Fundamental | The Licensed Water Sports Centre informs the users of Maritime Recreational Means of the safety rules prior to their use with a relevant demonstration (e.g. correct posture/seating, voluntary dismounting from the Maritime Recreational Means, etc.). |
| 6.3.30 | Fundamental | The Licensed Water Sports Centre informs the users of the Maritime Recreational Means about the actions to be taken as well as the procedure for collecting them in the event of their fall. |
| 6.3.31 | Fundamental | The Licensed Water Sports Centre has a case-by-case Emergency Needs and Situations Response Plan with regard to Maritime Recreational Means and their users and this is known by its staff. |
| 6.3.32 | Fundamental | The driver of the speedboat for towing Maritime Recreational Means avoids the deliberate fall of users from the towed means during service provision (e.g. extreme manoeuvres, etc.). |
| 6.3.33 | Fundamental | The driver of the speedboat for towing Maritime Recreational Means does not start unless all users are ready and prepared. |
| 6.3.34 | Fundamental | The driver of the speedboat for towing Maritime Recreational Means, or the user of motorised Maritime Recreational Means of the Licensed Water Sports Centre, pays particular attention and follows the necessary travel procedures in crowded areas with swimmers or other crafts. |
| 6.3.35 | Fundamental | The driver of the speedboat which tows the Maritime Recreational Means avoids towing the Maritime Recreational Means over the wake of nearby crafts. |
| 6.3.36 | Fundamental | The driver of the speedboat which tows the Maritime Recreational Means appropriately adapts (e.g. does not increase) the speed of the craft whilst turning to change course. |
| 6.3.37 | Fundamental | The towing of Maritime Recreational Means is not conducted in shallow waters and near the coastlines (as defined by the General Port Regulations). |
| 6.3.38 | Fundamental | The driver of the speedboat which tows the Maritime Recreational Means assesses the route effectively, based on the length of the tow rope and the obstacles. |
| 6.3.39 | Fundamental | The instruments of the speedboat, but also those of motorised means for rent (e.g. helm, gears, quick stop, etc.) respond effectively when used. |

S/N	 Criterion Characterisation

Description

| 6.3.40 |  | Fundamental | The Licensed Water Sports Centre takes into account the specific regulatory Functional Requirements (Annex I) for the leasing of the individual Maritime Recreational Means. |
| --- | --- | --- | --- |
|  | 6.4.Support equipment for |  | Maritime Recreational Means Services |
| Number |  | Criterion Characterisation | Description |
| 6.4.1 |  | Fundamental | The Licensed Water Sports Centre possesses means that can be used effectively for the transport of the injured and the provision of First Aid assistance (e.g. stretcher). |
| 6.4.2 |  | Fundamental | The Licensed Water Sports Centre has an appropriate First Aid box. |
| 6.4.3 |  | Fundamental | A portable artificial respiration device is available and the knowledge and ability to use it of the lessor or other employee of the Licensed Water Sports Centre is documented, and who is employed during all hours of company operation. |
| 6.4.4 |  | Fundamental | There is an Oxygen kit for First Aid. |
| 6.4.5 |  | Fundamental | The speedboats which tow the water ski and/or parasail and/or sea sled and/or water towable rings and/or any other similar means, provide life jackets for all passengers, as well as a ladder or platform astern for easy boarding and immediate collection of users of Maritime Recreational Means from the water. |
| 6.4.6 |  | Fundamental | The Licensed Water Sports Centre has, in sufficient quantities according to the available Maritime Recreational Means, life jackets of a bright colour in order to stand out from the water and for all sizes (children, adults, S, M, L, XL). |
| 6.4.7 |  | Fundamental | All Maritime Recreational Means that are entering into operation for the first time, as well as the support equipment (i.e. individual life jackets), carry CE markings and/or those of relevant certifications in accordance with applicable legislation. |
| 6.4.8 |  | Fundamental | The Licensed Water Sports Centre has a digital anemometer and/or weather instruments for collecting data (e.g. electronic applications) which |

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
|  |  | users can have access to, in particular those of towed flying Maritime Recreational Means and sailing boats. |
| 6.4.9 | Fundamental | The Licensed Water Sports Centre has a remote switching-off mechanism (control panel) for PWC-Jet skis, which operates effectively from a minimum distance of 1.000 metres. |
| 6.4.10 | Fundamental | The Licensed Water Sports Centre has a red flag for potential hoisting on the company's sail if the return of all Maritime Recreational Means, propelled by sail or cloth is necessary, to the launching point on the beach (as is the case with extraordinary weather conditions). |
| 6.4.11 | Fundamental | All members of the competent staff, whether on land or in the water, have wireless VHF communication devices or other appropriate and equally effective devices (if they move away from it) allowing for their immediate communication. |
| 6.4.12 | Fundamental | For the Maritime Recreational Means for which it is foreseen (e.g. kitesurf, fly board), the company provides, on a case-by-case basis, appropriate equipment for personal protection, e.g. helmets, and encourages their use. |
| 6.4.13 | Fundamental | The Licensed Water Sports Centre takes into account the more specific regulatory Competence and Characteristics Requirements for the Maritime Recreational Means equipment provided (Annex I) |

6.5.Support Facilities for Maritime Recreational Means

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.5.1 | Fundamental | The company provides its users with a controlled area and / or equipment (e.g. individual lockers, etc.) for the storage of personal belongings, during the use of the Maritime Recreational Means. |
| 6.5.2 | Fundamental | The company has sufficient space under cover, for restful (seated) waiting of the user before using the Maritime Recreational Means. |

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.5.3 | Fundamental | If the company has a legal platform, for the boarding of users on the Maritime Recreational Means, this is maintained in a suitable condition and has a non-slip surface. |

6.6Improving Services and Performance

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 6.6.1 | Fundamental | The Licensed Water Sports Centre should establish a mechanism and/or procedure for managing failures related to its activities and customer complaints. |
| 6.6.2 | Fundamental | The Licensed Water Sports Centre is equipped with a complaints box and a sufficient number of standardised forms. |

- | Optional (Basic) Requirements for Classification

of Certification

The criteria in this paragraph refer to optional "Basic" requirements which are assessed only if the Licensed Water Sports Centre wishes to obtain and/or maintain a higher qualification as to the level of certification for the classification of the Licensed Water Sports Centre. The said Characterisations (Gold, Silver, or Bronze) arise as a function of the Licensed Water Sports Centre's percentage compliance to these basic requirements, as mentioned below.

These criteria (Basic Requirements) are similarly audited, as are the mandatory (Fundamental Requirements), during the on-site audit of the Licensed Water Sports Centre.

- Organising the company's operational framework

- Staff & Responsibilities

- Performance & Provision of Water Sports

Optional criteria are specified for each of the six (6) categories for which there are mandatory (fundamental) requirements for certification.

- Support Equipment for Maritime Recreational Means Services

- Company facilities

- Improvement of Services

7.1.Organisation and operation

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.1.1 | Basic | The Licensed Water Sports Centre has sufficient documentation and records which (a) allow the organised and (b) document its effective operation, in terms of the quality of the services provided to its users / customers. |
| 7.1.2 | Basic | The Licensed Water Sports Centre manages its documents and records in such a way as to ensure their identification, updating them when necessary, to avoid unintended use of obsolete documents/records, and their current versions are available at the points of use. |
| 7.1.3 | Basic | The Licensed Water Sports Centre records, for all (motorised and non-motorised) maritime recreational means which it rents, the systematic monitoring/evaluation of their quality. |
| 7.1.4 | Basic | The Licensed Water Sports Centre records all available life jackets, with reference to their size (children's, adults, S, M, L, XL) as well as any comments about them. |
| 7.1.5 | Basic | The Licensed Water Sports Centre records all the other available equipment for the safety and protection of users with regard to the performance of the Maritime Recreational Means (e.g. helmets, communication devices, First Aid materials and means, etc.). |

7.2.Staff and Responsibilities

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.2.1. | Basic | For the provision of Kite Surfing, the Licensed Water Sports Centre has an instructor whose competence is documented by a relevant certificate from an official specialised body. |
| 7.2.2. | Basic | The Licensed Water Sports Centre describes the tasks, duties and areas of responsibility of the staff. |

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.2.3. | Basic | The training / briefing of staff on the company's policy and procedures on health and safety matters in the performance of the work is documented. |
| 7.2.4. | Basic | The minimum qualifications required (e.g. work experience, skills, etc.) per role are determined. |
| 7.2.5. | Basic | Confirmation of the understanding of the duties and responsibilities of staff is documented. |
| 7.2.6. | Basic | The regular assessment of the staff's response with regard to the quality of their work is documented and conducted by a person with relevant professional competence. |
| 7.2.7. | Basic | The Licensed Water Sports Centre documents the availability of an adequate and sufficient number of staff for the provision of its services throughout the duration of its operation. |

7.3.Performance of Maritime Recreational Means Services

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.3.1. | Basic | While the Maritime Recreational Means are being towed by a speedboat, apart from the qualified driver (according to par. 6.2.9) there is also a second person on board who is a member of the staff of the company with relevant training. |
| 7.3.2. | Basic | The company maintains a record of the data of users of motorised equipment e.g. jet ski, for which documented knowledge of how to handle them is required. |
| 7.3.3 | Basic | The Licensed Water Sports Centre has identified a demarcated sea area/arena within which the Maritime Recreational Means are allowed to operate (particularly for motorised Maritime Recreational Means, e.g.Jet ski, etc.). |

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.3.4 | Basic | The correct functioning of the boat used as a rescue boat is documented. |
| 7.3.5 | Basic | The company has and follows a safe approach procedure for the collection of users in the event of their fall from the Maritime Recreational Means. |
| 7.3.6 | Basic | The lessor or the employee is a holder of a CPR certificate. |
| 7.3.7 | Basic | The Licensed Water Sports Centre has defined and documents (e.g. sketch of the post) the manned point for monitoring the Maritime Recreational Means. |
| 7.3.8 | Basic | During rental of maritime recreational means, the monitoring post is supported by a competent, qualified person who is also authorised by the Licensed Water Sports Centre. |
| 7.3.9 | Basic | The Licensed Water Sports Centre has appropriate staff able to communicate matters of pricing policy and safety rules of the Maritime Recreational Means, adequately and effectively, to all users (e.g. with foreign persons). |
| 7.3.10 | Basic | The Licensed Water Sports Centre has set out defined positions and has means of effectively safeguarding the Maritime Recreational Means (e.g. closed circuit cameras, paid private security, etc.). |
| 7.3.11 | Basic | The Licensed Water Sports Centre has installed appropriate and specified buoys for the boundaries of the bathers' area, near the waterway and the operating area of its crafts. |
| 7.3.12 | Basic | Every user (irrespective of age) of a towed, motorised and non-motorised Maritime Recreational Means, signs a consent form for their participation in the activity. |
| 7.3.13 | Basic | The Licensed Water Sports Centre takes care to provide visual (or other suitable) information on the safety rules of the Maritime Recreational Means (as well as signals for the towed Maritime Recreational Means) for specific user groups (e.g. children, etc.) when using them. |
| 7.3.14 | Basic | The Licensed Water Sports Centre has a physician (e.g. workplace doctor) or access to a physician (appropriate medical specialist) for the provision of first aid, available during all hours of its operation. |

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.3.15 | Basic | The Licensed Water Sports Centre has access or takes care to provide easy approach for ambulances (or other rescue means) in the case the relative need arises for a user of Maritime Recreational Means. |
| 7.3.16 | Basic | The Licensed Water Sports Centre carries out regular checks (tests) for the effective communication between its members during full activity hours. |
| 7.3.17 | Basic | The Licensed Water Sports Centre carries out a practice effective response to an emergency situation, on an annual basis, with the involvement of all its competent staff. |
| 7.3.18 | Basic | The Licensed Water Sports Centre informs the users of the towed Maritime Recreational Means about hand signals. |
| 7.3.19 | Basic | The company takes care to maintain the staff involved in the provision of Maritime Recreational means (in particular operators of the towing speedboats) in an optimal physical condition for the performance of their duties (e.g. work hours, appropriate nutrition, rest, etc.). |
| 7.3.20 | Basic | The temporary storage of spare fuel for the company's motorised equipment takes place in a controlled area and is inaccessible to users. |

7.4.Support Equipment for Maritime Recreational Means Services

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.4.1 | Basic | The Licensed Water Sports Centre documents the regular monitoring during its period of operation, with regard to the continuous sufficiency of available First Aid materials and means. |
| 7.4.2 | Basic | A CPR Face Shield is available. |
| 7.4.3 | Basic | The company takes care of the cleaning, but also the maintenance of the quality of the life jackets after each use, with the appropriate means to this end. |

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.4.4 | Basic | The Licensed Water Sports Centre has the necessary equipment for the effective observation of customers, users of Maritime Recreational Means (e.g. binoculars, etc.). |
| 7.4.5 | Basic | For the Maritime Recreational Means that are being used simultaneously by two or more users in close proximity (e.g. a sea sled), the company also provides, on a case-by-case basis, the suitable personal protection equipment, e.g. helmets, and encourages their use. |

7.5.Support Facilities for Maritime Recreational Means

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.5.1 | Basic | The company provides its users with dressing rooms or showers for their proper preparation before and/or after using Maritime Recreational Means. |
| 7.5.2 | Basic | The company provides free access to clean toilets to accommodate the needs of users. |
| 7.5.3 | Basic | The company has a clean and tidy area for reception and waiting of users. |

7.6.Improving Services and Performance

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.6.1 | Basic | The Licensed Water Sports Centre proceeds in the evaluation and appraisal of the corrective actions in order to establish a relevant programme to avoid the recurrence of the same problems/failures. |
| 7.6.2 | Basic | The Licensed Water Sports Centre delegates responsibilities for the implementation of decisions for improving its services. |

| Number | Criterion Characterisation | Description |
| --- | --- | --- |
| 7.6.3 | Basic | The Licensed Water Sports Centre provides its users with the means (e.g. website, mobile application, etc.) for pre-booking rental services of Maritime Recreational Means. |
| 7.6.4 | Basic | The Licensed Water Sports Centre provides its users with ancillary actions that may upgrade the service without affecting its level of safety (e.g. photographs of the users, etc.). |
| 7.6.5 | Basic | The company hangs, in a prominent position, the "Charter"/policy of the entitlements provided by the same and anticipated by the user during the use of Maritime Recreational Means. |

## |Management of Complaints / Appeals / Grievances

8.1.Complaints

These pertain to expressions of dissatisfaction which are related to customer service matters and originate from the audited Licensed Water Sports Centre or other interested party. Examples include:

- delays in handling the audits,

- behavioural problems of Certification Body staff,

- issues related to the operation of the Licensed Water Sports Centre and are directly related to the scope of its certification.

8.2Appeals

The submission of appeals is related to matters of doubt on the part of the parties involved in the independent and objective judgement of the staff who carried out the audit, the technical competence of the staff involved in the audits, their reliability, etc.

8.3.Handling Complaints Related to Audit Requirements

Such complaints are submitted in writing to TAH and must be accompanied by relevant documentation (if not provided in the first instance by the interested party, TAH requests that it is submitted in the form of additional information).

For the investigation and handling, a team is formed consisting of persons with sufficient technical training (related to the subject under investigation).The team has at least three members and within this, external parties can participate on a consulting basis. Depending on the subject under investigation, an opinion or assistance from a relevant public authority may be requested.

8.4.Third Party Grievance

This pertains to grievances from interested parties, who are not connected to the audited Licensed Water Sports Centre or to TAH.

TAH takes care to gather, as much as possible, all information that substantiates the grievance and cross-references it with the findings of the most recent audit. TAH may decide:

- To contact the customer and / or interested party to receive data relating to the grievance, or

- Carry out the audit again to ascertain the correctness of the results of the initial audit or the allegations of the grievance

In the case in which the allegations of the grievance are found to be correct, and depending on the severity of the omission, TAH shall decide upon the corrective actions to be implemented by the Licensed Water Sports Centre or it shall proceed to revoke the certificate.

In any case where third party action is discovered, which harms the prestige of the audit or certification body, TAH shall investigate the necessity to impose sanctions by any legal means.

8.5.Documentation and Monitoring

All actions implemented in the context of handling all kinds of complaint / appeals / grievances are documented.
As a minimum, the following are identified:

- Details of implementation.

- The person(s) responsible for implementation.

- The implementation time.

- The person responsible for the verification of the sufficiency and effectiveness of the actions.

## |Revocation of Certification

Certification may be revoked if at least one of the following applies:

- Financial debts to TAH

- Failure to comply with the terms of the customer's agreement with TAH, both in terms of financial information and other requirements governing certification.

- The audit demonstrates the existence of serious non-conformities that compromise the safety of the services provided by the customer for the recipient, or the prestige of TAH.

- Customer's refusal or reluctance to comply with new requirements resulting from applicable legislation or amendments to the requirements of the Protocol which are decided by the Certification Body.

In any case where TAH considers that at least one of the above reasons exists, it shall inform the Licensed Water Sports Centre in writing at least 30 days before the revocation of the certificate, stating:

- the reasons for which it intends to carry out this action,

- the legislative and regulatory framework on the basis of which the claim to revoke certification is documented,

- any actions that can be implemented by the Licensed Water Sports Centre to avoid the revocation (if any, at the discretion of TAH).

The Licensed Water Sports Centre may submit its appeal towards the allegations of TAH.The appeals are examined by the Integrity Committee of the certification body which may, in the context of an investigation, obtain legal or other advice as required on a case-by-case basis.

Appeals can not be submitted if 30 days have elapsed since the notification to the Licensed Water Sports Centre of the intention to revoke certification.

Acceptance of the Licensed Water Sports Centre's appeal by TAH is done in writing and within no more than 20 days after its submission.

The results of the TAH’s examination of the appeal are communicated in writing to the submitter.

In the case of a positive result, e.g. acceptance of the objection, the certification body is not required to take any other action beyond written notification of the decision to the client.

In the case where the certification body maintains its original position, e.g. it rejects the allegations and evidence accompanying the appeal, then the certification body revokes the certification and:

- informs relevant interested parties / parties involved as required at the discretion of TAH,

- announces the revocation to all the media and / or networks in which it is active

- the Licensed Water Sports Centre is obliged to return the original certificate to TAH within 7 days.

## |Ownership and Management of the Certification Mark

- The right to use the Certification Mark for Certification of Licensed Water Sports Centres is valid only for the company audited by the Certification Body

- The use of the mark cannot be transferred by the Company to a third party or to a successor thereof (e.g. change of name of the Company).

- The mark must be legible, meet the specifications set out in this Protocol and be clearly distinguishable. The company is not entitled to make any kind of change without prior notice to TÜV AUSTRIA HELLAS.

- The company has the obligation to use the Certification Mark in such a way that it does not confuse or mislead the public about the type of field that has been inspected / evaluated by TÜV AUSTRIA HELLAS.

- In the event of claims against the Certification Body due to the Company’s use of the Evaluation Mark in a manner that is arbitrary, illegal and contrary to the provisions of this Protocol, the Company is obliged to exempt the Certification Body from any third party claims. The same applies in the case of third party claims against the Certification Body for assertions made by the Company for advertising purposes and that do not correspond to the terms of use of the Evaluation Mark, or confuse or mislead as to the type of field inspected / evaluated by TÜV AUSTRIA HELLAS.

- The Customer does not make or allow any misleading statement regarding its certification.

- Upon suspension or revocation of certification, it discontinues the use of all advertising media that contain a reference to the certification.

- Revise all promotional literature when the scope of certification has been reduced or modified.

- It does not imply that certification applies to activities that do not fall within the scope of certification.

- It does not use its certification in such a way that it will bring the Certification Body into disrepute, thus losing public confidence.

10.1.Expiry of Right to Use

The Licensed Water Sports Centre's right to use the mark and hold the Certificate ceases with immediate action if:

- The Licensed Water Sports Centre does not divulge to TAH any significant changes to the structure of the company, which have a direct impact on certification (e.g. change in legal form, headquarters of Principal),

- the result of the renewal audit does not justify maintaining the validity of the certificate,

- does not pay TAH's fee within the agreed time,

The Licensed Water Sports Centre's right to use the mark and to retain the Certificate also ceases immediately when the Licensed Water Sports Centre uses the mark in a manner contrary to the provisions of the agreement. TAH has the right, in the event that the terms of this article are not satisfied, to remove the Certificate and cancel it.

Upon expiry of the right to use, the Licensed Water Sports Centre is obliged to return the Certificate to TAH.

## |Scheme Safeguarding

11.1.Reliability

TAH undertakes the obligation to provide only the services that fall within the scope of its activity and takes care to ensure the necessary conditions for the proper provision of these services.

Both TAH and its representatives do not express certainty, in any way, regarding the achievement of results which are doubtful prior to the completion of the provision of the service, that is to say, the audit.

11.2.Independence / Impartiality / Integrity

All those involved in the audit procedure on the part of the Certification Body must meet the following conditions:

- Do not exercise a similar activity to that of the auditee and / or not engage in a similar activity of a third party.

- Are not involved in any way in the design, manufacture, sale, installation, maintenance of the equipment used by the Licensed Water Sports Centre for the provision of the audited service.

In general, the Certification Body and its staff should not be linked to activities which may come into conflict with the independence of their judgement and their integrity.

All interested parties must have access to the services of the Certification Body without undue financial or other conditions. Both the audit procedure and the other procedures relating to the overall operation of TAH must be carried out in a non-discriminatory manner.

The staff of TAH declare the preservation of their independence, integrity and impartiality in the performance of their duties, as well as their assurance regarding the confidentiality of the information that comes to their attention in writing. The written commitment of the staff is renewed annually whereas for newly recruited staff it is performed before they take up their duties.

11.3.Confidentiality

It is strictly forbidden to use information that comes to the attention of TAH in the exercise of its activity for its own benefit or for the benefit of third parties.

In cases in which TAH considers that there are serious reasons for using the customer's information in any way, it must have secured the written consent of the customer in advance.

The confidentiality obligation excludes information that is contrary to the applicable legislation, as well as cases where TAH is called upon by a judicial authority to assist.

11.4Integrity Committee

The Integrity Committee shall be composed of at least four members from the Certification Body, the Safe Water Sports Organisation and the interested party. The representatives of the bodies involved in the committee are appointed in writing by the body they represent and their appointment is valid for three years.

The Integrity Committee shall convene at least once a year to examine the compatibility of the operation of the Certification Body with the requirements of this Protocol.

The decisions of the committee shall be taken in a framework of objectivity and impartiality and shall be communicated to TAH, as well as to all the parties involved.

11.5Internal Audit

TAH is obliged to perform internal audits with a frequency of at least once a year to ensure the quality and appropriateness of the services provided.

The internal audit is carried out in such a way that it includes all its activities (internal operation and service provision), while the persons carrying out the internal audit may be present when conducting an audit.

## |Auditor Requirements

Personnel working for TAH shall observe the rules of independence, impartiality, objectivity and confidentiality mentioned in the preceding paragraph of this Protocol.

TAH's auditors in order to be considered capable of carrying out audits on the basis of the requirements of this Protocol shall have the following qualifications:.

- Be already recognised inspectors in at least one Management System

- Have attended a training seminar on the subject of the audit conducted by TAH,

- Be aware of the relevant legislative framework

- Have participated in at least 2 audits,

- Be familiar with the principles of the audit

- Be aware of the Audit Protocol

A prerequisite for recognition is the successful completion of written tests and on-site test auditing.

12.1.Auditor Training

All of the auditors working with the certification body attend initial induction training and this addresses the following:

- Auditor's duties

- Current legislation on the activity being carried out

- Function and requirements of this standard.

TAH is obliged to stay informed of any changes in the requirements (legislative or other) and to organise timely relevant training of its inspectors. The need for training of inspectors may arise:

- Following a revision of this Protocol

- After identifying serious shortcomings in the performance of audits

- From the requirement to upgrade the knowledge and skills of auditors in order to be in line with the current technological developments adopted by TAH.

The choice of training method depends on the subject of the training, whether there is a legislative requirement for a recognised training program, the availability (in terms of time) of an in-house instructor.

The certification body maintains an up-to-date file with full training details for all collaborating auditors.

12.2.Methodology for the Approval of Inspectors

### Initiating Collaboration

The certification body gathers documentary evidence of past experience in the subject (if any) and the details pertaining to the collaborator’s education and / or training.
The certification body subsequently:

- evaluates the prospective collaborator’s file,

- organises their theoretical and practical training,

- proceeds to the final evaluation and approval of the prospective collaborator, provided that all the necessary conditions are met.

### Monitoring Collaboration

The auditor's file is evaluated annually by TAH in order to continue the collaboration.

#### |Annex 1:  Conventional legislative requirements per Maritime Recreational Means

This Annex lists all conventional legislative requirements relating to all of the reported Maritime Recreational Means mentioned in the General Port Regulations.

These are classified in two main categories for the requirements of this Water Sports Certified Quality Audit and Certification Scheme. These pertain to:

- 1st Category: Motorised Maritime Recreational Means, and to

- 2nd Category: Non-Motorised Maritime Recreational Means

Specifically, for all Maritime Recreational Means that correspond to each of the above two categories, it relates to the set of corresponding 'General Competence and Characteristics Requirements for Maritime Recreational Means' as well as to the 'General Functional Requirements of Maritime Recreational Means'.

In addition, for each Maritime Recreational Means separately, the individual 'Competence & Characteristics' as well as 'Functional' conventional legislative requirements are further specified.

# 1st Category of Maritime Recreational Means

- | Motorised Maritime Recreational Means

Conventional legislative requirements for Motorised Maritime Recreational Means are separated below between those:

- Towed by Motorised Crafts (Towed Maritime Recreational Means) and

- Move autonomously (Self-propelled).

## 13.1 Maritime Recreational Means Towed by Motorised Crafts

General Competence & Characteristics Requirements for Maritime Recreational Means

| 1 | The use of an individual life-belt is mandatory for each user and has such features (such as, for example, smooth and soft texture, it is not inflatable, has no additional accessories other than those specified by the manufacturer) so as to provide maximum possible protection during its use, e.g. accidental injury in the event of a fall, keeping the user on the surface of the sea, etc. |
| --- | --- |
| 2 | The speedboat used for towing is in each case that which is stipulated in the Licensed Water Sports Centre's license, with the necessary power for the successful (safe) towing of the Maritime Recreational Means with its passengers at any given time. |
| 3 | The tow rope of the Maritime Recreational means has appropriate characteristics (e.g. diameter, length, etc.) and is in accordance with construction specifications for the Maritime Recreational Means, and can stay afloat in the sea. |
| 4 | The point of attachment of the tow rope(s) to the craft has all of the strength and stability characteristics for safe towing. |
| 5 | The main equipment used for the towed Maritime Recreational Means (e.g. waterskis, parachutes, sleds, etc.) does not bear any damages which may cause unintentional injury to the user. |

General Functional Requirements of Maritime Recreational Means

| 6 | The driver of the speedboat used for towing the Maritime Recreational Means holds the necessary qualifications and has the same responsibilities as the owners of speedboats. |
| --- | --- |
| 7 | The driver of a speedboat used for towing the practitioner of the Maritime Recreational Means takes care to avoid an accident, following routes free of stresses (seafarming-cages, etc.) or floating |

General Functional Requirements of Maritime Recreational Means

|  | sea structures (platforms, etc.) as well as away from the routes of other nearby boats. |
| --- | --- |
| 8 | The driver of the speedboat shall ensure that Maritime Recreational Means are towed at an appropriate distance from the furthest point where bathers usually swim, e.g. more than one hundred (100) metres outside of the floating buoys which mark the boundaries to which bathers usually reach in bathing facilities or two hundred (200) metres from the usual point where bathers reach by swimming in bathing areas not marked by floating buoys or as otherwise specified each time by applicable legislation. |
| 9 | The towing of Maritime Recreational Means is adapted to weather conditions and is not carried out in marginal adverse weather conditions (e.g. waves and winds above 5 Beaufort, before sunrise or after sunset, etc.) |

| 13.1. 1. Water skiing |  |
| --- | --- |
| Number | Competence & Characteristics Requirements |
| 1 | The tow rope has a length of up to 23 metres, with a diameter of 8 mm. |
| 2 | The waterskis and cleats have the necessary quality characteristics for safe / seamless towing of skiers, depending on their weight and age. |
|  | Functional Requirements for Maritime Recreational Means |
| 3 | The Licensed Water Sports Centre ensures that whilst towing one skier, no other Maritime Recreational Means are being towed simultaneously. |

| 13.1. 2. Parasailing or Parachuting |  |
| --- | --- |
| Number | Competence & Characteristics Requirements |
| 1 | The craft used for towing has the appropriate characteristics of length (more than 5 metres), weight (minimum of 450kg) and power (minimum of 125HP outboard, 140HP inboard or 160HP stern drive) for successful (safe) towing and execution of the parachute, in accordance with the type characteristics of the parachute, as defined in the applicable legislation. |

| 13.1. 2. Parasailing or Parachuting |  |
| --- | --- |
| 2 | The speedboat used for towing the parachute has a suitable mechanism (winch) for lifting, either from its inbuilt platform or from a legal floating platform. |
| 3 | In order to tow a two-seater JAMBO-type parachute, the craft has a minimum weight of 600kg and a minimum engine power of 140HP outboard, 160HP inboard or 200HP |
| 4 | The equipment for parasailing satisfies the transport of a maximum of two users. |
| 5 | The tow rope shall be up to eighty (80) metres in length and have a diameter of at least twelve (12) millimetres for single seater parachutes and fourteen (14) millimetres for two-seater parachutes and capable of responding to the towing forces depending on the type of parachute (one-seater, two-seater). |

| 13.1. 2. | Parasailing or Parachuting |
| --- | --- |
| Number | Functional Requirements for Maritime Recreational Means |
| 6 | The authorised, qualified staff for towing parachutes will ensure the safe provision of the service and the meticulous checking of the tow rope as well as the parachute attachments used to fasten it to the individual, while also checking the rope attachment points on the speedboat. |
| 7 | The use of parachutes is not permitted for persons under the age of 14 and this is clearly communicated to potential users. |
| 8 | The lessors have an anemometer at the lifting point of the parachute (platform-speedboat). |
| 9 | The Licensed Water Sports Centre ensures that no other Maritime Recreational Means is towed whilst the parachute is being towed, by the same craft. |
| 10 | The Licensed Water Sports Centre ensures that the launching or returning of the craft towing the parachute does not take place within a distance of 200 metres from other passing crafts on either side of its route. |
| 11 | The ascent / descent of the parachute is conducted in accordance with the particularities of the area and with the aim of avoiding accidents: |

| 13.1. 2. | Parasailing or Parachuting |
| --- | --- |
|  | - from a floating platform as per Article 28 concerning parasailing, - from a speedboat which has a built-in platform and winch, - from the lessor's platform which has been installed near the coast and for which there is a license from the local Authority. |
| 12 | If it is required, the transportation (to - from) of persons wishing to practice/enjoy parasailing, from the lessor's post on the shore to the launching points (1) and (2) of the previous requirements, is permitted by non-motorised maritime means or motorised maritime means, excluding the personal water craft and HOVER CRAFT, and provided these are travelling in a demarcated waterway and do not reach speed greater than five (5) knots. |

| 13.1. 3. Sea Sled (Water Towable Tube) |  |
| --- | --- |
| Number | Functional Requirements for Maritime Recreational Means |
| 1 | The towing of a sea sled is carried out at a speed of up to twenty-five (25) knots maximum. |
| 2 | The maximum number of persons on the sea sled is equal to the number of positions of the sled for which corresponding pairs of handles are available. |
| 3 | The Licensed Water Sports Centre makes sure to tow only one sea sled at a time. |

| 13.1. 4. Inflatable Maritime Recreational Means (e.g. water towable ring, etc.) |  |
| --- | --- |
| Number | Functional Requirements for Maritime Recreational Means |
| 1 | The towing of a maritime inflatable is conducted at a speed of up to twenty-five (25) knots maximum. |
| 2 | From each speedboat it is possible to tow up to four single-seater or two two-seater or one three-seater or one four-seater or one five-seater or one six-seater, inflatable maritime recreational means. |

| 13.1. 4. Inflatable Maritime Recreational Means (e.g. water towable ring, etc.) |  |
| --- | --- |
| 3 | The maximum number of persons on the sea sled is equal to the number of positions of the sled for which corresponding pairs of handles are available. |

| 13.1. 5. Flyfish - Flying Banana |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The maximum speed of use of the flyfish must not be greater than eighteen (18) knots. |
| 2 | A maximum of six persons board the flyfish 6 Pro/ FlyingBanana, with a maximum weight of the persons at seven hundred (700) kilograms.At the flyfish 2 Pro, a maximum of two (2) people board, with a maximum weight of the persons at three hundred (300) kilograms.A maximum of two (2) people board the flyfish small, with a maximum weight of the persons at one hundred and forty (140) kilograms. |
| 3 | Only a single flyfish is allowed to be towed by each speedboat. |

13.2.Self-propelled Maritime Recreational Means

For the requirements of the Water Sports Certified Quality Audit and Certification Scheme, the self-propelled Maritime Recreational Means are distinguished as those with:

A. High-Horsepower Engine B. Low-Horsepower Engine

13.2Α. High-Horsepower Motorised Maritime Recreational Means

|  | General Functional Requirements of Maritime Recreational Means |
| --- | --- |
| 1 | The high-horsepower motorised Maritime Recreational Means ensures that they are not used when adverse weather conditions prevail. |

General Functional Requirements of Maritime Recreational Means

| 2 | The lessor of high-horsepower motorised Maritime Recreational Means, ensures that these are not used before sunrise or after sunset and, in any case, when visibility is limited. |
| --- | --- |
| 3 | The lessor of High-Horsepower motorised Maritime Recreational Means ensures that these are boarded by as many persons as are defined by the manufacturer for the specific type of Maritime Recreational Means. |
| 4 | The lessor of High-Horsepower motorised Maritime Recreational Means ensures that they are used outside of floating buoys, which mark the limits to which bathers usually reach by swimming in the bathing facilities. |

| 13.2A. 1. Personal water crafts (SeaBeetle, SeaBike, Jet-Surf, Jet Ski etc.) |  |
| --- | --- |
| Number | Competence & Characteristics Requirements |
| 1 | The lessor of personal water crafts of any type and name (SEA BEETLE, SEA BIKE, JET-SURF, JET SKI, etc.) ensures that they carry the necessary equipment specified by their manufacturer (danger signals and fire extinguishers) or other of similar effectiveness (e.g. two (2) handheld flares, one (1) smoke-flare and one (1) small powder fire extinguisher) |
| 2 | The lessor of personal water crafts of any type and name ensures that they have an automatic switching-off (QUICK STOP) system to switch-off the engine when their operator falls into the sea. |
| 3 | The lessor of personal water crafts of any type and name ensures that they are not used when a fault has been found or their engines have broken down. |
| 4 | The lessor of personal water crafts of any type and name, ensures that they do not carry additional parts that may cause injury to their users, such as external propellers. |

| 13.2A. 2. Personal water crafts (SeaBeetle, SeaBike, Jet-Surf, Jet Ski etc.) |  |
| --- | --- |
| Number | Functional Requirements for Maritime Recreational Means |

| 13.2A. 2. Personal water crafts (SeaBeetle, SeaBike, Jet-Surf, Jet Ski etc.) |  |
| --- | --- |
| 5 | The lessor of personal water crafts of any type or name ensures that they do not approach rocky sea areas. |
| 6 | The lessor of personal water crafts of any type or name ensures that their handling - irrespective of the manufacture type or the speed they can reach - is done by persons who possess a speedboat or personal water craft driving license. |
| 7 | The lessor of personal water crafts of any type or name ensures that the Maritime Recreational Means are not towed by personal water crafts (e.g. for water skiing or inflatable Maritime Recreational Means, etc.). |
| 8 | The lessor of personal water crafts of any type or name ensures that they are used: - at a distance of more than two hundred (200) metres from the outside of the floating buoys indicating the limits to which bathers swim in bathing facilities. - at a distance of more than three hundred (300) metres from the usual spot where bathers swim in sea areas not marked by floating buoys. - with a maximum speed of five (5) knots and at a distance of more than five hundred (500) metres from the coastline in areas where there are no bathers. |
| 9 | The lessor of personal water crafts of any type or name ensures that they are used within the areas defined in the rental licenses granted and for which visual contact with the post is ensured. |
| 10 | The lessor of personal water crafts of any type or name ensures that their launch / return is carried out from and to the specified points of the company's operating license. |
| 11 | The lessor of personal water crafts of any type and/or name ensures that the transportation (to - from) of persons, wishing to practice/enjoy these, from the lessor's post on the shore to their launching (positions) points, is carried out by non-motorised maritime means or motorised maritime means excluding the personal water craft and HOVER CRAFT, provided these are travelling in demarcated waterways and do not reach speeds greater than five (5) knots. |

| 13.2A. 3. Flyboard- Jetovator- X Jetpack |  |
| --- | --- |
| S/N | Competence & Characteristics Requirements |
| 1 | The lessor of the Maritime Recreational Means takes care to check the normal and secure connection of the lifting equipment with the personal water craft, in accordance with the manufacturer's provisions. |
| 2 | The lessor of the Maritime Recreational Means ensures that the personal water craft has an automatic switching-off (quick stop) system in the event that the legal driver falls off. |

| 13.2A. 4. Flyboard- Jetovator- X Jetpack |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 3 | The lessor of the Maritime Recreational Means ensures that in cases where he is not the driver of the personal water craft, he is replaced by his legally employed employee, who will possess a speedboat driving license. |
| 4 | The lessor of the Maritime Recreational Means ensures that the user of the Flyboard - Jetovator, X Jetpack, wears a protective waterproof suit and protective helmet, in addition to the individual life jacket. |
| 5 | The lessor of the Maritime Recreational Means ensures that a demarcated waterway is used when launching and returning to and from the coastline and at a speed of less than 5 knots, as defined by the General Port Regulations. |
| 6 | The lessor ensures to rent the Maritime Recreational means to persons over the age of 18 who know how to swim. |
| 7 | The lessor of the Maritime Recreational Means ensures that the Maritime Recreational Means are not used with a wind force of more than 4 Beaufort (with wave height above 1 metre). |
| 8 | The lessor of the Maritime Recreational Means ensures that the Maritime Recreational Means are not used in appropriate areas with a sufficient depth not less than 4 metres. |
| 9 | The lessor of the Maritime Recreational Means ensures that they are used: o at a distance of more than two (2) nautical miles from the coast |

| 13.2A. 4. Flyboard- Jetovator- X Jetpack |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
|  | - at a distance of more than two hundred (200) metres from the outside of the floating buoys indicating the limits to which bathers swim in bathing facilities. - at a distance of more than three hundred (300) metres from the usual spot where bathers swim in sea areas not marked by floating buoys. - at a distance of more than five hundred (500) metres from the coastline in areas where there are no bathers and/or floating buoys. - at a distance of more than 150 metres from buoys marking underwater activities - at a distance of more than 100 metres from anchored ships, floating platforms, rocks and other obstacles - outside of ports, marinas and fishing shelters and at a distance of more than 1 nautical mile around their entrances - at a speed of less than 18 knots and at a height of less than 4 metres |
| 10 | The lessor of the Maritime Recreational Means documents the use of the personal water craft (including the spare one he may possess) to which the lifting equipment is connected and which are listed on the rental license. |

| 13.2A. 5. Hovercraft |  |
| --- | --- |
| S/N | Competence & Characteristics Requirements |
| 1 | The Hovercraft driver documents his competence to use it by way of a speedboat driving license as well as an instructor's license issued by the manufacturer. |
| 2 | The legal owner of the craft ensures that it is registered and equipped with all the provisions stipulated in the sailing license. |

| 13.2A. 5. Hovercraft |  |
| --- | --- |
|  | Functional Requirements for Maritime Recreational Means |
| 3 | The driver of the Hovercraft ensures its exclusive travel in the sea |
| 4 | The Hovercraft driver ensures that it is used: - at a distance of more than two hundred (200) metres from the outside of the floating buoys indicating the limits to which bathers swim in bathing facilities. - at a distance of more than three hundred (300) metres from the usual spot where bathers swim in sea areas not marked by floating buoys. - with a maximum speed of five (5) knots and at a distance of more than five hundred (500) metres from the coastline in areas where there are no bathers. |
| 5 | The lessor of the Hovercraft ensures that the launching and return of the craft is conducted to and from the following points: - floating platform, as that provided in the General Port Regulations. - a special anchoring buoy (anchorage), as prescribed in the General Port Regulations - any point of the sea that is a greater distance from the limits (distances) of the previous requirement. In the event that the above points are shorter distances than the foreseen distances, the launch / return of the rented personal water craft is performed at a speed of less than five (5) knots. |
| 6 | The lessor of the Hover Craft ensures that the transportation (to - from) of persons, wishing to practice/entertain themselves with the craft, from the lessor's post on the shore to their launching (positions) points, is carried out by non-motorised maritime means or motorised maritime means excluding the personal water craft and HOVER CRAFT, provided this is travelling in demarcated waterways and does not increase speed greater than five (5) knots. |

| 13.2A. 6. Motor boats |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The lessor of the Motor Boat ensures that the craft is used by persons over the age of sixteen (16), if it is not operated by him. |
| 2 | The legal owner of the craft ensures that it is registered and equipped with all the provisions stipulated in the sailing license. |
| 3 | The lessor of the Motor Boat ensures that it is used: - at a distance greater than one hundred (100) metres from the outside of the floating buoys which indicate the limits to which bathers usually swim in bathing facilities and in any case greater than that defined by the Competent Authority through the Committee of the General Port Regulations and mentioned in the license. - at a distance greater than two hundred (200) metres from the usual spot where bathers swim in sea areas not marked with floating buoys. - at a maximum speed of five (5) knots and more than two hundred (200) metres from the coastline in the areas where there are no bathers or less, if defined by the competent authority through the Committee of the General Port Regulations and is mentioned in the license. |
| 4 | The lessor of the motor boat ensures that launching and return from / to the coast is conducted within the demarcated waterway of the General Port Regulations and at a maximum of five (5) knots. |

| 13.2A. 7. Oxoon / Oxoon Jet / Oxon New Generation / Hydroxoon F1 Maritime Recreational Means |  |
| --- | --- |
| S/N | Competence & Characteristics Requirements |
| 1 | The lessor of the Maritime Recreational Means (OXOON JET and OXOON NEW GENERATION) ensures that they bear the CE marking according to Directive 94/25/EC |

| 13.2A. 7. Oxoon / Oxoon Jet / Oxon New Generation / Hydroxoon F1 Maritime Recreational Means |  |
| --- | --- |
| 2 | The lessor of the Maritime Recreational Means, OXOON/OXOON JET/OXOON NEW GENERATION, ensures that they carry the necessary equipment in a watertight container, including two (2) handheld flares, one (1) smoke-flare and one (1) small powder fire extinguisher. |
|  | Functional Requirements for Maritime Recreational Means |
| 3 | The lessor of Maritime Recreational Means (OXOON JET and OXOON NEW GENERATION) ensures that they are operated only by a person over the age of eighteen (18) and 14 for the HYDROXOON F1. |
| 4 | The lessor of the Maritime Recreational Means (OXOON JET and OXOON NEW GENERATION) ensures that they are not boarded by more than three (3) persons or one (1) person for the HYDROXOON F1, respectively. |
| 5 | The lessor of the Maritime Recreational Means ensures that the launch and return to / from the coast is conducted through the foreseen demarcated waterway. |
| 6 | The lessor of the Maritime Recreational means ensures that persons boarding them are over the age of twelve (12). |
| 7 | The lessor of the Maritime Recreational Means makes sure that they are used: - at a distance greater than fifty (50) metres from the outside of the floating buoys which indicate the limits to which bathers usually swim in bathing facilities. - at a distance greater than one hundred (100) metres from the usual spot where bathers swim in sea areas not marked by floating buoys. - up to a maximum distance of (500) metres from the coastline and two nautical miles from the launching point. |
| 8 | The lessor takes care to prevent the use of Maritime Recreational Means in rocky sea areas or outside of sheltered bays and coves. |
| 9 | The lessor takes care to prevent the Maritime Recreational Means from being used when wind force in the wider sea area is greater than four (4) Beaufort. |
| 10 | The lessor of the Maritime Recreational Means ensures that their driver is equipped with a speedboat or personal water craft driving license in the case when the craft has an engine of more than 15HP maximum continuous horsepower. |

| 13.2A. 8. Hawaii, Hawaii GT, Adventurer and Adventurer GT |  |
| --- | --- |
| S/N | Competence & Characteristics Requirements |
| 1 | The lessor of the Maritime Recreational Means ensures that they carry necessary equipment in a watertight container, including two (2) handheld flares, one (1) smoke-flare and one (1) small powder fire extinguisher. |
|  | Functional Requirements for Maritime Recreational Means |
| 2 | The lessor of the Maritime Recreational Means ensures that they are operated only by persons over the age of eighteen (18) and that persons under the age of twelve (12) do not board. |
| 3 | The lessor of the Maritime Recreational Means ensures that the launch and return to / from the coast is conducted through the foreseen demarcated waterway. |
| 4 | The lessor of the Maritime Recreational Means makes sure that they are used: - at a distance greater than fifty (50) metres from the outside of the floating buoys which indicate the limits to which bathers usually swim in bathing facilities. - at a distance greater than one hundred (100) metres from the usual spot where bathers swim in sea areas not marked by floating buoys. - up to a maximum distance of (500) metres from the coastline and two nautical miles from the launching point. |
| 5 | The lessor takes care to prevent the use of Maritime Recreational Means in rocky sea areas or outside of sheltered bays and coves. |
| 6 | The lessor takes care to prevent the Maritime Recreational Means from being used when wind force in the wider sea area is greater than four (4) Beaufort. |
| 7 | The lessor of the Maritime Recreational Means ensures that their driver is equipped with a speedboat or personal water craft driving license in the case when the craft has an engine of more than 15HP maximum continuous horsepower. |

13.2Β. Low-Horsepower Motorised Maritime Recreational Means

General Functional Requirements of Maritime Recreational Means

| 1 | The lessor of low-horsepower Maritime Recreational Means ensures that they are not used when adverse weather conditions prevail. |
| --- | --- |
| 2 | The lessor of low-horsepower motorised Maritime Recreational Means ensures that these are not used before sunrise or after sunset and, in any case, when visibility is limited. |
| 3 | The lessor of low-horsepower motored Maritime Recreational Means ensures that they carry as many persons as the manufacturer stipulates for the specific type of Maritime Recreational Means. |
| 4 | The lessor of low-horsepower motorised Maritime Recreational Means ensures that its use does not take place more than five hundred (500) metres from the coastline. |
| 5 | The lessor of low-horsepower motorised Maritime Recreational Means informs the user to ensure that proper care and attention is shown so as not to disturb bathers. |

| 13.2B. 1. Aqua Scooter / Sea - Doo Sea Scooter |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The lessor ensures that the Maritime Recreational Means is used by individuals over the age of fourteen (14) for the Aqua Scooter and 16 for the Sea - Doo Sea Scooter. |
| 2 | The lessor ensures that the Maritime Recreational Means are used by persons wearing a suitable plastic cap on their head. |
| 3 | The lessor ensures that if the Maritime Recreational Means is used for underwater activity, the provisions of the General Port Regulations are complies with. |

| 13.2B. 2. Mini Cruiser |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The lessor ensures that the Maritime Recreational Means is used by persons over the age of eighteen (18). |

| 2 | The lessor ensures that only one (1) person boards the Maritime Recreational Means. |
| --- | --- |

| 13.2B. 3. Bbq Donut |  |
| --- | --- |
| S/N | Competence & Characteristics Requirements |
| 1 | The lessor of the Maritime Recreational Means ensures that it bears the CE marking in accordance with Directive 94/25/EC, as well as individual life jackets for the passengers, a smoke-flare and fire extinguisher. |
|  | Functional Requirements for Maritime Recreational Means |
| 2 | The lessor ensures that the Maritime Recreational Means is used by persons over the age of sixteen (16) or otherwise, if adults board who know how to swim. |
| 3 | The lessor supervises that there is no eating or drinking on the Maritime Recreational Means. |
| 4 | The lessor ensure that no more than ten (10) persons with a total weight of less than 1500kilograms board the Maritime Recreational Means, including any additional supplies they may carry. |
| 5 | The lessor ensures that the power of the outboard engine of the Maritime Recreational Means does not exceed 5 HP or 3.68 kW. |
| 6 | The lessor takes care to prevent the Maritime Recreational Means from being used when wind force in the wider sea area is greater than 4 Beaufort. |
| 7 | The lessor of the Maritime Recreational Means ensures that movement is carried out at a distance greater than (100) metres from the usual point where bathers reach by swimming and within the floating buoys, where these are obligatory. |
| 8 | The lessor of the Maritime Recreational Means ensures that users use it within the foreseen demarcated waterway when launching and returning to and from the coastline. |

| 13.2B. 4. Funboat - Chaiselounger |  |
| --- | --- |
| S/N | Competence & Characteristics Requirements |
| 1 | The lessor of Maritime Recreational Means, ensures that the craft has mandatory supplies such as - individual life jackets for passengers, - one smoke-flare, - oars, - anchor with rope - a lifesaver buoy with a 15-metre rope, - a foam or CO2 or powder fire extinguisher, - a radio with batteries, - three flares, - a folding knife and - a First Aid kit. |
|  | Functional Requirements for Maritime Recreational Means |
| 2 | The lessor of the Maritime Recreational Means ensures that the battery used by the craft is not of the sealed type and has a main supply disconnect switch. |
| 3 | The lessor ensures that the total weight of supplies and passengers does not exceed - one hundred and forty five point five (145.5) kilograms on the FUN BOAT, and - three hundred and eighteen (318) kilograms on the CHAISE LOUNGER. |
| 4 | The lessor shall ensure that the outboard electric engine has its propeller in a protective cage and its power does not exceed six (6) kW. |
| 5 | The lessor of the Maritime Recreational Means ensures that it travels at a distance greater than (200) metres from the usual point where bathers reach by swimming and within the floating buoys, where these are obligatory. |

| 13.2B. 4. Funboat - Chaiselounger |  |
| --- | --- |
| 6 | The lessor ensures that the Maritime Recreational Means is used by persons over the age of sixteen (16) or otherwise, if adults board who know how to swim. |
| 7 | The lessor of the Maritime Recreational Means ensures that no more than two (2) passengers board the FUN BOAT and four (4) on the CHAISE LOUNGER. |

| 13.2B. 5. PowerSurf XS |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The lessor ensures that the Maritime Recreational Means is used by persons over the age of fourteen (14). |
| 2 | The lessor ensures that the passenger wears a personal life-belt when using the Maritime Recreational Means. |
| 3 | The lessor takes care to prevent the use of Maritime Recreational Means in rocky sea areas or outside of sheltered bays and coves. |

13.2B. 6. Waterbuggy

S/N	Functional Requirements of Maritime Recreational Means

1

The lessor ensures that the Maritime Recreational Means is used by persons

over the age of sixteen (16) or otherwise, if adults board who know how to swim.

2

The lessor ensures to prevent the use of Maritime Recreational Means when the

wind force in the wider sea area is greater than 4 Beaufort and the wave height exceeds 0.3 metres.

3

The lessor takes care to prevent the use of Maritime Recreational Means outside

of sheltered bays or coves.

4

The lessor ensures that the power of the outboard engine of the Maritime Recreational Means does not

exceed 11.1 kW and there is a protective cage at the propulsion end of the engine.

| 13.2B. 6. Waterbuggy |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 5 | The lessor ensure that no more than two (2) persons with a total weight of less than 170kilograms board the Maritime Recreational Means, including any additional supplies they may carry. |
| 6 | The lessor ensures that the user of the Maritime Recreational Means travels at a maximum speed of up to four (4) knots. |
| 7 | The lessor of the Maritime Recreational Means ensures that it travels at a distance greater than (200) metres from the usual point where bathers reach by swimming and within the floating buoys, where these are obligatory. |
| 8 | The lessor of Maritime Recreational Means ensures that it carries one (1) fire extinguisher, one (1) smoke-flare, two (2) flares, one (1) anchor with a rope, and one five (5) metre tow rope. |
| 9 | The lessor of the Maritime Recreational Means ensures that throughout its use the passengers all wear life jackets with a whistle. |
| 10 | The lessor of Maritime Recreational Means ensures that users are informed of the use of the demarcated waterway when launching and returning to and from the coastline. |

| 13.2B. 7. Sea Spi single and Sea Spi twin carries |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The lessor ensures that the Maritime Recreational Means is used by persons over the age of sixteen (16) or otherwise, if adults board who know how to swim. |
| 2 | The lessor takes care to prevent the use of Maritime Recreational Means outside of sheltered bays or coves. |
| 3 | The lessor of the Maritime Recreational Means ensures that the battery used by the craft is not of the sealed type and has a main supply disconnect switch. |

| 13.2B. 7. Sea Spi single and Sea Spi twin carries |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 4 | The lessor ensures that the power of the outboard engine of the Maritime Recreational Means does not exceed six (6) kW and there is a protective cage at the end of the propeller. |
| 5 | The lessor ensures that the total weight of supplies and passengers does not exceed - one hundred and forty (140) kilograms in the SEA SPI single, and - two hundred and seventy-three (273) kilograms in the SEA SPI twin carries. |
| 6 | The lessor of the Maritime Recreational Means ensures that it travels at a distance greater than (200) metres from the usual point where bathers reach by swimming and within the floating buoys, where these are obligatory. |
| 7 | The lessor of the Maritime Recreational Means ensures that the boarding of one (01) person on the SEA SPI single and a maximum of two (02) persons on the SEA SPI twin carries, is observed. |
| 8 | The lessor of Maritime Recreational Means, ensures that the craft has mandatory supplies such as - individual life jackets for passengers, - one smoke-flare, - oars, - anchor with rope - a lifesaver buoy with a 15-metre rope, - a foam or CO2 or powder fire extinguisher, - a radio with batteries, - three flares, - a folding knife and - a First Aid kit. |

| 13.2B. 8. Catamaran with Electric Propulsion |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The lessor of the Maritime Recreational Means ensures that it travels at a distance greater than (200) metres from the usual point where bathers reach by swimming and within the floating buoys, where these are obligatory. |
| 2 | The lessor of the Maritime Recreational Means ensures that the battery used by the craft is not of the sealed type and has a main supply disconnect switch. |

| 13.2B. 8. Catamaran with Electric Propulsion |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 3 | The lessor ensures that the power of the outboard engine of the Maritime Recreational Means does not exceed four (4) kW and there is a protective cage at the propeller end. |
| 4 | The lessor ensures that the total weight of supplies and passengers on the Maritime Recreational Means does not exceed two hundred and fifty (250) kilograms. |
| 5 | The lessor ensures that the Maritime Recreational Means is used by persons over the age of sixteen (16) or otherwise, if adults board who know how to swim. |
| 6 | The lessor ensures that the Maritime Recreational Means carries a maximum of three (3) persons. |
| 7 | The lessor of Maritime Recreational Means, ensures that the craft has mandatory supplies such as - individual life jackets for passengers, - one smoke-flare, - oars, - anchor with rope - a lifesaver buoy with a 15-metre rope, - a foam or CO2 or powder fire extinguisher, - a radio with batteries, - three flares, - a folding knife and a First Aid kit. |

2nd Category of Maritime Recreational Means

- | Non-Motorised Maritime Recreational Means

14.1 Wind-powered Maritime Recreational Means

|  | General Competence & Characteristics Requirements for Maritime Recreational Means |
| --- | --- |
| 1 | The use of an individual life-belt is mandatory for each user and has such features (such as, for example, smooth and soft texture, it is not inflatable, has no additional accessories other than those specified by the manufacturer) so as to provide maximum possible protection during its use, e.g. accidental injury in the event of a fall, keeping the user on the surface of the sea, etc. |

General Functional Requirements of Maritime Recreational Means

| 2 | The movement of wind-powered Maritime Recreational Means is carried out at a distance greater than two hundred and 200 metres from the usual point where bathers reach by swimming, in sea areas not marked by floating buoys or outside of floating buoys where these exist. |
| --- | --- |
| 3 | The movement of wind-powered Maritime Recreational Means is adapted to weather conditions and is not conducted in marginal adverse weather conditions (e.g. waves and winds above 5 Beaufort - or if otherwise defined by legislation - before sunrise or after sunset, etc.) |

| 14.1. 1. Windsurf (WS) |  |
| --- | --- |
| Number | Functional Requirements for Maritime Recreational Means |
| 1 | When the WS is launched from the shore or during their return, the passenger exercises proper care and attention to avoid disturbing bathers |
| 2 | The WS does not travel in marginal adverse weather conditions with a wind force of over 6 Beaufort |

| 14.1. 2. Kitesurf |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The use of the kitesurf shall be carried out in sea areas which are (a) free from rocks, rock islands, reefs, casted nets or other obstacles; (b) within a reasonable distance from airports and especially from the assumed landing and take-off lines of |

| 14.1. 2. Kitesurf |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
|  | airplanes; (c) at least one (01) nautical mile from port, marina and fishing shelter entrances. |
| 2 | Users of the kitesurf must carry the following safety equipment, including: - Helmet - Suitable belt, with hook - Suitable knife for cutting the kite ropes, in case of emergency. - Suitable life jacket. - Rapid kite release system, for instant release (LEASE SYSTEM). - Kite push-away system (QUICK RELEASE). |
| 3 | The kitesurf operator takes care to prevent unintentional take-off of the kite when the equipment is located on the beach. |
| 4 | The operator of the kitesurf makes sure that it is not used: - Within highly-populated beaches. - When adverse weather conditions with the risk of lightning prevail. - Beyond one (1) nautical mile from the coasts. - When the wind direction is towards the open sea and there is no supervision by a person with a motorised maritime means, to retrieve the operator in case of emergency. - When the wind force is over six (06) Beaufort and there is no supervision by a person with a valid lifeguard certificate, as well as a motorised maritime means, to retrieve the practitioner in case of emergency. |
| 5 | For the rental of kite surfing equipment, the following conditions are ensured: 1.Existence of a demarcated waterway, at least 100 metres wide and at least one hundred (100) metres long. The marking of the waterway boundaries must be effected by appropriate red buoys, without these being connected between them with a rope. |

| 14.1. 2. Kitesurf |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
|  | - The flying of the kite of the kitesurf is carried out from a beach area on the extension of the aforementioned sea waterway, with dimensions one hundred by twenty (100 x 20) metres. - Clear information exists for bathers on the boundaries of the kitesurf take-off area, in the Greek and English language, and to encourage them to take preventive measures to avoid accidental injury. - A weather station is displayed at the lessor's area, showing the direction and force of the wind. - The simultaneous renting of the kitesurf with other permitted Maritime Recreational Means, is not allowed, with the exception of the windsurf. |
| 6 | The flying of the kite is only permitted when there are no physical or other obstacles within a one hundred (100) metre radius of the user at the time of take-off. |

| 14.1. 3. Catamaran |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The maximum number of passengers on a rented CATAMARAN is as indicated in the sailing license. |
| 2 | The CATAMARANs are registered in Small Boat Registration Books (ΒΕΜΣ) and have their names and serial numbers, of their registration in these books, written on both sides |
| 3 | The rented CATAMARANs sail within a maximum distance of one thousand (1000) metres from the rental post on the shore so that they are under the constant and immediate supervision of their lessors. |

| 14.1. 4. Sailing boats |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |

| 1 | Sailing boats are registered in the Small Boat Registration Books (ΒΕΜΣ) and have their names and serial numbers, of their registration in these books, written on both sides |
| --- | --- |
| 2 | Sailing boats comply, in terms of life-saving firefighting equipment, the number of passengers and the distance from the coastline as specified in the sailing license. |
| 3 | The rented sailboats sail within a maximum distance of one thousand (1000) metres from the rental post on the shore so that they are under the constant and immediate supervision of their lessors. |

14.2.Maritime Recreational Means Propelled by Paddle Wheel

|  | General Functional Requirements of Maritime Recreational Means |
| --- | --- |
| 1 | The travel of Non-Motorised Maritime Recreational Means is adapted to the weather conditions and is not conducted in marginal adverse weather conditions (e.g. waves and winds above 5 Beaufort - or unless otherwise provided by legislation - before sunrise or after sunset etc.) |

| 14.2. 1. Pedal Boats |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The use of pedal boats by persons under the age of fourteen (14) is only possible if an adult person is also on it. |
| 2 | The use of pedal boats is allowed at a maximum distance of five hundred (500) metres from the coastline. |
| 3 | Pedal boats carry as many people as their manufacture type stipulates. |
| 4 | The operator of the pedal boat must be careful to avoid disturbing bathers. |
| 5 | The passengers of the EXPLORER type of pedal boats carry a personal life-belt |

| 14.2. 2. Seabuggy |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The use of the Seabuggy (SB) is allowed at a maximum distance of five hundred (500) metres from the coastline. |
| 2 | Only one person may board the SB, its driver. |
| 3 | The driver of the SB must be careful to avoid disturbing bathers. |

| 14.2. 3. Aqua Paddle Boat |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The use of the Aqua Paddle Boat by people under the age of ten (10) is only allowed if an adult is in on board. |
| 2 | The movement of the Aqua Paddle Boat is allowed up to a maximum wind force of 4 Beaufort in the wider sea area. |
| 3 | The movement of the Aqua Paddle Boat is allowed in areas without rocks and within sheltered bays or coves. |
| 4 | The movement of the Aqua Paddle Boat is allowed in a sea area deeper than fifteen (15) centimetres and up to a maximum of three (03) metres |
| 5 | The maximum permitted speed of a Aqua Paddle Boat is two (2) km/h |
| 6 | The passengers of the Aqua Paddle Boats should be in a seated position and wear a life jacket with a whistle, throughout its use. |
| 7 | The number of passengers on board an Aqua Paddle Boat is as stipulated by its manufacture type. |
| 8 | The movement of a rented Aqua Paddle Boat is obligatorily conducted in sea areas where there is a visual contact with the renting post. |
| 9 | The use of an Aqua Paddle Boat is only for recreation and not for other use (e.g. transport of loads, persons, rescue means, fishing, etc.). |

| 14.2. 4. Acrylicraft |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The Acrylicraft carries a maximum of two (2) persons, one of whom is over the age of eighteen (18). |
| 2 | The boat obligatorily travels without the use of a motor or sail. |
| 3 | The movement of the Acrylicraft is (a) away from rocky sea areas, and (b) in areas where it is easy to swim to shore in case of emergency. |
| 4 | On the boat, during travel, two individual life belts are carried for the use of the passengers |
| 5 | The movement of the Acrylicraft is allowed up to a maximum of one hundred (100) metres from the coastline but away from bathers and not closer than 50 metres from them. |
| 6 | The use of the Acrylicraft is only for recreation and not for other use (e.g. transport of loads, persons, rescue means, fishing, etc.). |

14.3.Rowing Maritime Recreational Means

General Functional Requirements of Maritime Recreational Means

| 1 | The movement of Rowing Maritime Recreational Means is adapted to the weather conditions and is not carried out in marginal adverse weather conditions (e.g. waves and winds above 5 Beaufort - or unless otherwise provided by legislation - before sunrise or after sunset etc.) |
| --- | --- |
| 2 | The movement of the Rowing Maritime Recreational Means is allowed only if a maximum of as many persons as stipulated by the manufacturer, for this specific type and use of Maritime Recreational Means, board. |
| 3 | The movement of the Rowing Maritime Recreational Means is allowed up to a maximum distance of five hundred (500) metres from the coastline. |
| 4 | The operator of the Rowing Maritime Recreational Means ensures to avoid disturbing bathers. |

| 14.3. 1. Canoe - Kayak |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The use of Canoes is allowed by people over the age of fourteen (14) and sixteen (16) for Kayaks, respectively. |
| 2 | The passengers of the Kayak have an individual life belt. |

| 14.3. 2. Stand-Up Paddle Board (SUP) |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The use of the SUP is permitted by individuals over the age of fourteen (14) - Its use by people who do not have an individual life belt. - Its movement using a sail. |
| 2 | The movement / use of the SUP is only allowed if one (1) person is on it. |

14.4.Inflatable Water Toys & Games

|  | General Competence & Characteristics Requirements for Maritime Recreational Means |
| --- | --- |
| 1 | The lessor of the inflatable water toys ensures the correct installation (fitting, assembling and fixing) of the parts comprising the recreational means, so that this operates and is used in a safe condition by those renting them. |

General Functional Requirements of Maritime Recreational Means

| 2 | The lessor of inflatable water toys ensures that these are not used before sunrise or after sunset and, in any case, when the visibility is limited. |
| --- | --- |
| 3 | The lessor of inflatable water toys ensures that they are placed in a sea area with a suitable depth as appropriate and in accordance with that defined by the legislator or the manufacturer. |

General Functional Requirements of Maritime Recreational Means

| 4 | The lessor of the inflatable water toys ensures that in the area in which they are deployed, there is no simultaneous exercise of any other activity (boating, fishing, etc.). |
| --- | --- |
| 5 | The lessor of the inflatable water toys ensures that the area in which they are used, is far away from rocky sea areas or areas with a rocky seabed. |

| 14.4. 1.Waterblob |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 1 | The lessor does not allow the use of the waterblob by persons under the age of twelve (12). |
| 2 | The use of the waterblob takes place outside of the sea areas that are usually for ship transit, as well as outside of ports, marinas and fishing shelters or even around them.The appropriate distance is defined by the decision of the Port Authority, depending on the specifics of the area, and in any case not less than five hundred (500) metres. |
| 3 | The lessor does not allow the use of the waterblob when adverse weather conditions prevail (wind force of five (5) on the Beaufort scale and above). |
| 4 | The waterblob is used in sea areas with a minimum depth of three (3) metres. |
| 5 | The lessor of the waterblob ensures: - there are no objects (such as poles, buoys, etc.) at least six (6) metres from the recreation point. - It is not installed and operated indoors or when there is an obstacle over the head of the jumper at a height of less than five (5) metres. - Not to install Maritime Recreational Means within organised bathing facilities |
| 6 | The lessor takes care to prevent underwater swimming beneath the maritime means throughout its use. |

| 14.4. 1.Waterblob |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 7 | Throughout the use of this maritime means, the passengers wear a life jacket and a helmet. |
| 8 | The difference in body weight between the person jumping (jumper) from a diving board or similar installation and the person catapulted (blobber) must be less than thirty (30) kilograms for children and sixty (60) kilograms for adults, who have understood the proper use of the means and are healthy and able-bodied. |
| 9 | The height of the jump must not exceed five (05) metres from the surface of the sea. |

| 14.4. 2. Inflatable Water Park |  |
| --- | --- |
| S/N | Competence & Characteristics Requirements |
| 1 | All of the individual parts that together form the recreational water park meet the requirements of Directive 2001/95/EC, which was incorporated into our law by the Joint Ministerial Decision.Z3/2810/14-12-2004 (Government Gazette 1885 Β) as well as the harmonised standards EN 15649 (parts 1-7) which have been included in the national standardisation system and are valid as national standards ELOT EN under the exclusive responsibility of their producer. |
| 2 | The deployment of the means does not exceed forty (40) metres, both along the length of the coastline, as well as from the coast out to sea. |

| 14.4. 2. Inflatable Water Park |  |
| --- | --- |
| S/N | Functional Requirements for Maritime Recreational Means |
| 3 | The lessor ensures that the area in which the recreational water park is set up is marked by the placement of appropriate distinct floating buoys (e.g. yellow colour and spherical shape) located four (4) metres apart. |

| 4 | The lessor ensures that the maximum number of renters / users of the means of the recreational water park is always consistent with that specified in the manufacturer's specifications. |
| --- | --- |
| 5 | Throughout the rental of the recreational water park, the lessor ensures to recruit and retain a qualified lifeguard in the sea area, in accordance with the provisions of the PD.23/2000 (A18). (The existence of a lifeguard pedestal and special equipment, in accordance with the Annexes to the aforementioned PD, is not required) |
| 6 | The Inflatable Water Park operates in sea areas with a minimum depth of fifty (50) centimetres. |
| 7 | The lessor does not allow the operation of Inflatable Water Parks when adverse weather conditions prevail (wind force of four (04) Beaufort and above). |
| 8 | The lessor ensures that one hour after sunset, the Maritime Recreational Means are removed from the sea area and are re-positioned not earlier than one hour before sunrise. |

| 14.4. 3. Water Polo Goal Posts, Basketball Hoops, Volleyball Nets |  |
| --- | --- |
| S/N | Competence & Characteristics Requirements |
| 1 | The Maritime Recreational Means (Water Polo Goal Posts, Basketball Hoops, Volleyball Nets) are CE marked or, if they are not required so, the lessor has a manufacturer's attestation that they do not pose risks to users during their normal use. |
| 2 | The deployment of the Maritime Recreational Means along the coastline follows the provisions of the attestation / act of the body which grants the space on the seashore. |
|  | Functional Requirements for Maritime Recreational Means |
| 3 | The lessor ensures that the area in which the recreational water park is set up is marked by the placement of appropriate distinct floating buoys (e.g. yellow colour and spherical shape) located four (4) metres apart. |
| 4 | With the care of the lessor these means remain firm in the same position, during the course of the rental, by the placement of spherical |

| 14.4. 3. Water Polo Goal Posts, Basketball Hoops, Volleyball Nets |  |
| --- | --- |
|  | weights, placed under the seabed, at a depth of at least fifteen centimetres. |
| 5 | The lessor ensures that one hour after sunset, the Maritime Recreational Means are removed from the sea area and are re-positioned not earlier than one hour before sunrise. |
| 6 | Throughout the rental of Maritime Recreational Means, the lessor ensures to recruit and retain a qualified lifeguard in the sea area, in accordance with the provisions of the PD.23/2000 (A18). (The existence of a lifeguard pedestal and special equipment, in accordance with the Annexes to the aforementioned PD, is not required) |

- | Annex 2 :  Audit Forms

15.1	Audit Report

| 1.Basic Company Details |  |  |  |  |
| --- | --- | --- | --- | --- |
| Company Name |  |  |  |  |
| Full address (Street / number / Postal Code / City / Beach) |  |  |  |  |
| Lessor |  |  |  |  |
| Responsible by law |  |  |  |  |
| Rental License for Maritime Recreational Means |  |  |  |  |
| Licensing Port Authority |  |  |  |  |
| Company Description (number of employees, main activities, commercial activities, brief history) |  |  |  |  |
| 2.Inspection details |  |  |  |  |
| Date of Initial Audit |  |  | Date of Renewal Audit |  |
| Inspection duration (M/D) |  |  |  |  |
| Scope of Certification - Means for rent |  |  |  |  |
| Type of audit |  |  |  |  |
| Full Name of Auditor |  |  |  |  |

Audit Objectives

To define: the requirements for the operation of Licensed Water Sports Centres related to the safety of water sports users. It also includes the requirements for the adherence of the regulatory and legislative framework which pertains to the operation of the Licensed Water Sports Centre as well as the safe provision of their services to the users of Maritime Recreational Means. Secondary Purpose: to define the requirements for the documentation, the quality support and the management of data and details related to the operation of the Licensed Water Sports Centres. The audit was conducted on the basis of the relevant Certification Protocol for Licensed Water Sports Centres, of TÜV AUSTRIA HELLAS. The basic information documenting the result of the audit is included in this report, and in its entirety, in the inspection checklist and in the copies of documents and other evidence obtained from the audited Licensed Water Sports Centre.

Characterisation of results

1.Full Compliance

2.Deviation - the verification of the effectiveness of the corrective action is assessed in the next audit

3.Non-Compliance:Revocation with repeat audit / document submission

Audit Report of the Water Sports Certified Quality Protocol

| LIST OF DEVIATIONS / NON-CONFORMITIES |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | Criterion |  | Description of findings |  | Category | Determination of Cause and Corrective action |  | Evidence of N/C Rectification |  | Assessment / Verification of Corrective Action |
|  |  |  |  |  |  |  |  |  | Removal of Deviations |  |
| Location, Date: |  |  |  |  |  |  | Location, Date: |  | Location, Date: |  |
| Auditor (name, signature) |  |  |  | Company Representative (name, signature) |  |  | Company Representative (name, signature) |  | Lead Inspector (name, signature) |  |

Version 2 - January 2018 	Page 78 of 121

| 3.3 Auditor's Recommendation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Issue of Certificate |  |  | FOLLOWING |  | no other action |  |
|  | Renewal of Certificate |  |  |  |  | Rectification of any Non-Conformities with Documentation Submission |  |
|  | Revocation of Certification |  |  |  |  | Rectification of any Non-Conformities with Repeat Inspection |  |
| COMPLIANCE LEVEL |  | PLATINUM | GOLD |  |  | SILVER | BRONZE |

| Location, Date: |  |
| --- | --- |
| Auditor |  |
| Signature |  |

Other information:

During the validity of the certificate, the certified company / organisation has the obligation to inform the Certification Body of any changes it makes to its operation and consequently to its documentation. It should be stressed that the audit is a sample audit, a fact which means that there may be other non-conformities in addition to those that may have been recorded during the assessment audit. The audit result does not relieve the inspected company / organisation from the responsibility to adhere to and comply with the requirements of the protocol on the basis of which the Certificate was awarded.

The certification body or the auditor, under no circumstances do they substitute or replace the audit performed by the competent bodies of the Hellenic State.

The responsibility for adherence and appreciation of the conformity, to the relevant legislative and regulatory provisions lies, in any case, with the Company.

| CUSTOMER ACCEPTANCE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company Representative (Full Name) |  | Signature/Stamp |  | Period of Next Audit (Month and year) |  |

15.2 	Audit Checklist

| 1.Basic Company Details |  |  |  |  |
| --- | --- | --- | --- | --- |
| Company Name |  |  |  |  |
| Full address (Street / number / Postal Code / City / Beach) |  |  |  |  |
| Lessor |  |  |  |  |
| Responsible by law |  |  |  |  |
| Rental License for Maritime Recreational Means |  |  |  |  |
| Licensing Port Authority |  |  |  |  |
| Company Description (number of employees, main activities, commercial activities, brief history) |  |  |  |  |
| 2.Inspection details |  |  |  |  |
| Date of Initial Audit |  |  | Date of Renewal Audit |  |
| Inspection duration (M/D) |  |  |  |  |
| Scope of Certification - Means for rent |  |  |  |  |
| Type of audit |  |  |  |  |
| Full Name of Auditor |  |  |  |  |

Instructions for completing the checklist:

- The "Requirements" column determines the paragraph of the Protocol for which the audit is being conducted. The "Requirements" are distinguished between Mandatory (Fundamental) and Optional (Basic).

- The "Data - Examples" column records the objective data that were assessed during the audit (files, examples, etc.)

- Column "A" is completed with the result of the assessment of the data submitted to the auditor, as follows:

Characterisation of results

1.Fully Compliant

2.Deviation - Improvement point verification of the effectiveness of the corrective action is assessed in the next audit

3.Non-Compliance:Removal with monitoring audit

- A separate completed checklist is required for each audit

- The checklist forms an integral part of the overall audit documentation.Incomplete completion and / or non-completion, in whole or in part, constitutes a reason for the incompleteness of the audit file.

| LEGISLATIVE REQUIREMENTS: |  |
| --- | --- |
| Operating License, Facility Approval (when required): |  |
| Mandatory Legislation, Standards, Regulations, MinisterialDecisions, etc.: |  |
| Fire Safety Certificate: |  |
| Approval of Environmental Terms |  |
| Lifeguard: |  |

## Mandatory (Fundamental) audit criteria for certification

| Number | CRITERION 1:Organisation and operation | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.1.1 | The Licensed Water Sports Centre has a copy of the valid rental license available at the Licensed Water Sports Centre's post where services are provided, and in a prominent place. |  |  |
| 6.1.2 | The Licensed Water Sports Centre has insurance policies for each craft (and the "rescue" boat) and for the Maritime Recreational Means that fall under the specific legislative requirement, which are legally in its possession and which are used for the services it provides. The insurance policy covers third party liability insurance for death, personal injuries and material damage, in force. |  |  |
| 6.1.3 | The Licensed Water Sports Centre has licenses to sail in place for each craft (and the "rescue" boat). |  |  |
| 6.1.4 | The Licensed Water Sports Centre records by type (motorised and not) all available Maritime Recreational Means for rent (e.g. speedboat, jet ski, water ring, kitesurf, etc.), and verifies the correct recording of names and Small Boat Registration Books (ΒΕΜΣ) numbers (for any Maritime Recreational Means subject to such a requirement). |  |  |

| Number | CRITERION 2:Staff and Responsibilities | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.2.1. | The Licensed Water Sports Centre assigns the implementation of the work to staff with the necessary (special) training and qualifications as defined by the legal and regulatory framework, and also by the company policy. |  |  |
| 6.2.2 | If the rental license includes as maritime recreational means for rent, water skiing and/or parasailing and/or the sea sled and/or the water towable ring and/or any other which is similar to these and is towed by a speedboat, the responsible lessor possesses (a) a water skiing instructor's license of any category under the General Secretariat for Sports (GSS) and a speedboat driving license for at least two years or (b) is a graduate of the Faculty of Physical Education and Sports Science with a specialisation in water skiing and has, for at least one year, a speedboat driving license or (c) already holds a valid water skiing instructor's license, whose validity has been certified under the provisions of the General Port Regulations and its holder has previously practiced, under the authorisation of the Port Authority, the profession of lessor or has provided actual and active services to a lessor for maritime recreational means towed by a speedboat. |  |  |
| 6.2.3 | Subsequent to the previous audit point, provided that the Licensed Water Sports Centre has available assistant(s) to the above responsible lessor, this assistant(s) is occupied with the performance of water skiing and/or parasailing and/or the sea sled and/or the water towable ring and/or any other which is similar to that towed by a speedboat and |  |  |

| Number | CRITERION 2:Staff and Responsibilities | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
|  | for this purpose has been audited and either: (a) holds a water skiing instructor's license of any category under the General Secretariat for Sports (GSS) and a speedboat driving license for at least two years or (b) is a graduate of the Faculty of Physical Education and Sports Science with a specialisation in water skiing and has, for at least one year, a speedboat driving license or (c) already holds a valid water skiing instructor's license, whose validity has been certified under the provisions of the General Port Regulations and its holder has previously practiced, under the authorisation of the Port Authority, the profession of lessor or has provided actual and active services to a lessor for maritime recreational means towed by a speedboat. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.3.1 | The Licensed Water Sports Centre rents out only maritime recreational means, by type and number, which are included in the rental license. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.3.2 | While the Maritime Recreational Means are being towed by a speedboat, there is a second person on board besides the qualified driver (according to par. 6.2.9) for the continuous monitoring (supervision) of the practitioner, who has received the necessary briefing. |  |  |
| 6.3.3 | The Licensed Water Sports Centre documents the rental of a maximum of three personal water crafts at any given time, which it has in its possession and for which the legislative documents are available. |  |  |
| 6.3.4 | The company evaluates and documents the competence of users of motorised equipment, e.g. jet ski, for which knowledge of how to handle them is required before their use. |  |  |
| 6.3.5 | The Licensed Water Sports Centre has captured and monitors all the potential routes of all types of motorised maritime recreational means, for which (routes) it notifies users (regarding these and for the other Maritime Recreational Means). |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.3.6 | For the description of each potential route of the floating Maritime Recreational Means, parameters which may affect their motion, have been taken into account and are marked or recorded on a relevant sketch and are communicated to the users. |  |  |
| 6.3.7 | The lessor does not himself engage in a simultaneous activity (e.g. the use of seats or sun-beds or shading equipment, etc.), which may affect the safe provision of the services of the Licensed Water Sports Centre. |  |  |
| 6.3.8 | The lessor does not employ and does not allow the employment of his staff, whose duties are to support the Maritime Recreational Means services, in other commercial activities (e.g. rental of beach umbrellas, etc.). |  |  |
| 6.3.9 | The activities within the area of business activity of the Licensed Water Sports Centre (in and out of the sea), which may affect the safe provision of its services, have been taken into account and assessed adequately by the same (e.g. operation of another Licensed Water Sports Centre, anchorage, etc.). |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services |  |  |  |  |  |  | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.3.10 | The Licensed Water Sports Centre possesses a motor boat with features which allow it to be used as a rescue boat. |  |  |  |  |  |  |  |  |
| 6.3.11 | The Licensed Water Sports Centre does not rent out the rescue boat and/or the use of the speedboat, which serves as a rescue boat, for any other purpose other than the provision of rescue services. |  |  |  |  |  |  |  |  |
| 6.3.12 | The lessor or the employee documents their knowledge of Basic Life Support (primary assessment, CPR, rescue breaths). |  |  |  |  |  |  |  |  |
| 6.3.13 | The Licensed Water Sports Centre is supported by a lifeguard whose competence is documented. |  |  |  |  |  |  |  |  |
| 6.3.14 | The lessor, from the point where he is located can in fact reasonably observe as much as possible the rented recreational means, to provide assistance in case of danger. |  |  |  |  |  |  |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.3.15 | The Licensed Water Sports Centre does not allow renting to persons under the influence of alcohol or toxic substances. |  |  |
| 6.3.16 | The Licensed Water Sports Centre has posted a copy of the price list of the services at a suitable point of the post (e.g. on the shore), in the Greek and English language. |  |  |
| 6.3.17 | The Licensed Water Sports Centre has the capability to store all the Maritime Recreational Means after the end of the working day in order to ensure that they will not be used. |  |  |
| 6.3.18 | The boat, which may be used at any time as a rescue boat, is in a point and in a condition such that it can immediately provide rescue services to users of Maritime Recreational Means (e.g. maintenance, sufficient amount of fuel , etc.). |  |  |
| 6.3.19 | The Licensed Water Sports Centre has a clearly identified waterway as a means of transit of the maritime recreational means, during their travel from and to the launching point, where this is foreseen and required. |  |  |
| 6.3.20 | The Licensed Water Sports Centre has installed a special buoy (mooring) to be used by the lessor as a means of launching the motorised recreational means, provided this is stated on the license. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.3.21 | The Licensed Water Sports Centre has anchored its own private or specially chartered boat, to be used by the lessor as a means of launching the motorised recreational means, provided this is stated on the license. |  |  |
| 6.3.22 | Subsequent to the previous audit point, the boat has raised the anchorage signals prescribed by the International Regulations for the Prevention of Marine Collisions. |  |  |
| 6.3.23 | In the area of activity of the Licensed Water Sports Centre, the area of travel of the rented crafts and the bathers is distinguishable. |  |  |
| 6.3.24 | All Maritime Recreational Means are regularly checked and maintained by the Licensed Water Sports Centre without altering the features specified by the manufacturer. |  |  |
| 6.3.25 | All life jackets made available by the Licensed Water Sports Centre have a bright colour, except those used for water skiing. |  |  |
| 6.3.26 | The Licensed Water Sports Centre provides life jackets of appropriate size and shape to users of Maritime Recreational Means and checks their proper use by practitioners prior to commencement of the activity. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.3.27 | All passengers of Maritime Recreational Means wear life jackets throughout the duration of their use. The lessor and his assistant may be exempt from this obligation. |  |  |
| 6.3.28 | For users that are minors, the company has a consent form to participate in the activity. The form is signed by the custodian or the parent or the supervisor of the minor, or it simply accompanies the minor. In this the minor's ability to swim must be confirmed. |  |  |
| 6.3.29 | The Licensed Water Sports Centre informs the users of Maritime Recreational Means of the safety rules prior to their use with a relevant demonstration (e.g. correct posture/seating, voluntary dismounting from the Maritime Recreational Means, etc.). |  |  |
| 6.3.30 | The Licensed Water Sports Centre informs the users of the Maritime Recreational Means about the actions to be taken as well as the procedure for collecting them in the event of their fall. |  |  |
| 6.3.31 | The Licensed Water Sports Centre has a case-by-case Emergency Needs and Situations Response Plan with regard to Maritime Recreational Means and their users and this is known by its staff. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.3.32 | The driver of the speedboat for towing Maritime Recreational Means avoids the deliberate fall of users from the towed means during service provision (e.g. extreme manoeuvres, etc.). |  |  |
| 6.3.33 | The driver of the speedboat for towing Maritime Recreational Means does not start unless all users are ready and prepared. |  |  |
| 6.3.34 | The driver of the speedboat for towing Maritime Recreational Means, or the user of motorised Maritime Recreational Means of the Licensed Water Sports Centre, pays particular attention and follows the necessary travel procedures in crowded areas with swimmers or other crafts. |  |  |
| 6.3.35 | The driver of the speedboat which tows the Maritime Recreational Means avoids towing the Maritime Recreational Means over the wake of nearby crafts. |  |  |
| 6.3.36 | The driver of the speedboat which tows the Maritime Recreational Means appropriately adapts (e.g. does not increase) the speed of the craft whilst turning to change course. |  |  |
| 6.3.37 | The towing of Maritime Recreational Means is not conducted in shallow waters and near the coastlines (as defined by the General Port Regulations). |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.3.38 | The driver of the speedboat which tows the Maritime Recreational Means assesses the route effectively, based on the length of the tow rope and the obstacles. |  |  |
| 6.3.39 | The instruments of the speedboat, but also those of motorised means for rent (e.g. helm, gears, quick stop, etc.) respond effectively when used. |  |  |
| 6.3.40 | The Licensed Water Sports Centre takes into account the specific regulatory Functional Requirements (Annex I) for the leasing of the individual Maritime Recreational Means. |  |  |

| Number | CRITERION 4: Support Equipment for Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.4.1 | The Licensed Water Sports Centre possesses means that can be used effectively for the transport of the injured and the provision of First Aid assistance (e.g. stretcher). |  |  |
| 6.4.2 | The Licensed Water Sports Centre has an appropriate First Aid box. |  |  |
| 6.4.3 | A portable artificial respiration device is available and the knowledge and ability to use it of the lessor or other employee of the Licensed Water Sports Centre is documented, and who is employed during all hours of company operation. |  |  |
| 6.4.4 | There is an Oxygen kit for First Aid. |  |  |
| 6.4.5 | The speedboats which tow the water ski and/or parasail and/or sea sled and/or water towable rings and/or any other similar means, provide life jackets for all passengers, as well as a ladder or platform astern for easy boarding and immediate collection of users of Maritime Recreational Means from the water. |  |  |
| 6.4.6 | The Licensed Water Sports Centre has, in sufficient quantities according to the available Maritime Recreational Means, life jackets of a bright colour in order to stand out from the water and for all sizes (children, adults, S, M, L, XL). |  |  |

| Number | CRITERION 4: Support Equipment for Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.4.7 | All Maritime Recreational Means that are entering into operation for the first time, as well as the support equipment (i.e. individual life jackets), carry CE markings and/or those of relevant certifications in accordance with applicable legislation. |  |  |
| 6.4.8 | The Licensed Water Sports Centre has a digital anemometer and/or weather instruments for collecting data (e.g. electronic applications) which users can have access to, in particular those of towed flying Maritime Recreational Means and sailing boats. |  |  |
| 6.4.9 | The Licensed Water Sports Centre has a remote switching-off mechanism (control panel) for PWC-Jet skis, which operates effectively from a minimum distance of 1.000 metres. |  |  |
| 6.4.10 | The Licensed Water Sports Centre has a red flag for potential hoisting on the company's sail if the return of all Maritime Recreational Means, propelled by sail or cloth is necessary, to the launching point on the beach (as is the case of extraordinary weather conditions). |  |  |
| 6.4.11 | All members of the competent staff, whether on land or in the water, have wireless VHF communication devices or other appropriate and equally effective devices (if they move away from it) allowing for their immediate communication. |  |  |

| Number | CRITERION 4: Support Equipment for Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.4.12 | For the Maritime Recreational Means for which it is foreseen (e.g. kitesurf, flyboard), the company provides, on a case-by-case basis, appropriate equipment for personal protection, e.g. helmets, and encourages their use. |  |  |
| 6.4.13 | The Licensed Water Sports Centre takes into account the more specific regulatory Competence and Characteristics Requirements for the Maritime Recreational Means equipment provided (Annex I) |  |  |

| Number | CRITERION 5:Support Facilities for Maritime Recreational Means | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.5.1 | The company provides its users with a controlled area and / or equipment (e.g. individual lockers, etc.) for the storage of personal belongings, during the use of the Maritime Recreational Means. |  |  |
| 6.5.2 | The company has sufficient space under cover, for restful (seated) waiting of the user before using the Maritime Recreational Means. |  |  |

| Number | CRITERION 5:Support Facilities for Maritime Recreational Means | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.5.3 | If the company has a legal platform, for the boarding of users on the Maritime Recreational Means, this is maintained in a suitable condition and has a non-slip surface. |  |  |

| Number | CRITERION 6: Improving Services and Performance | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 6.6.1 | The Licensed Water Sports Centre should establish a mechanism and/or procedure for managing failures related to its activities and customer complaints. |  |  |

| Number | CRITERION 6: Improving Services and Performance |  |  |  |  |  |  | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.6.2 | The Licensed Water Sports Centre is equipped with a complaints box and a sufficient number of standardised forms. | - | - | - | - | - | - |  |  |

Optional (Basic) Requirements for Classification of Certification

| Number | CRITERION 1:Organisation and operation | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.1.1 | The Licensed Water Sports Centre has sufficient documentation and records which (a) allow the organised and (b) document its effective operation, in terms of the quality of the services provided to its users/customers. |  |  |
| 7.1.2 | The Licensed Water Sports Centre manages its documents and records in such a way as to ensure their identification, updating them when necessary, to avoid unintended use of obsolete documents/records, and their current versions are available at the points of use. |  |  |
| 7.1.3 | The Licensed Water Sports Centre records, for all (motorised and non-motorised) maritime recreational means which it rents, the systematic monitoring/evaluation of their quality. |  |  |
| 7.1.4 | The Licensed Water Sports Centre records all available life jackets, with reference to their size (children, adults, S, M, L, XL), as well as any comments about them. |  |  |

| Number | CRITERION 1:Organisation and operation | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.1.5 | The Licensed Water Sports Centre records all the other available equipment for the safety and protection of users with regard to the performance of the Maritime Recreational Means (e.g. helmets, communication devices, First Aid materials and means, etc.). |  |  |

| Number | CRITERION 2:Staff and Responsibilities | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.2.1 | For the provision of Kite Surfing, the Licensed Water Sports Centre has an instructor whose competence is documented by a relevant certificate from an official specialised body. |  |  |
| 7.2.2 | The Licensed Water Sports Centre describes the tasks, duties and areas of responsibility of the staff. |  |  |

| 7.2.3 | The training / briefing of staff on the company's policy and procedures on health and safety matters in the performance of the work is documented. |  |  |
| --- | --- | --- | --- |
| 7.2.4 | The minimum qualifications required (e.g. work experience, skills, etc.) per role are determined. |  |  |
| 7.2.5 | Confirmation of the understanding of the duties and responsibilities of staff is documented. |  |  |
| 7.2.6 | The regular assessment of the staff's response with regard to the quality of their work is documented and conducted by a person with relevant professional competence. |  |  |
| 7.2.7 | The Licensed Water Sports Centre documents the availability of an adequate and sufficient number of staff for the provision of its services throughout the duration of its operation. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.3.1 | While the Maritime Recreational Means are being towed by a speedboat, apart from the qualified driver (according to par. 6.2.9) there is also a second person on board who is a member of the staff of the company with relevant training. |  |  |
| 7.3.2 | The company maintains a record of the data of users of motorised equipment e.g. jet ski, for which documented knowledge of how to handle them is required. |  |  |
| 7.3.3 | The Licensed Water Sports Centre has identified a demarcated sea area/arena within which the Maritime Recreational Means are allowed to operate (particularly for motorised Maritime Recreational Means, e.g. Jet ski, etc.). |  |  |
| 7.3.4 | The correct functioning of the boat used as a rescue boat is documented. |  |  |
| 7.3.5 | The company has and follows a safe approach procedure for the collection of users in the event of their fall from the Maritime Recreational Means. |  |  |
| 7.3.6 | The lessor or the employee is the holder of a CPR certificate. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.3.7 | The Licensed Water Sports Centre has defined and documents (e.g. sketch of the post) the manned point for monitoring the Maritime Recreational Means. |  |  |
| 7.3.8 | During rental of maritime recreational means, the monitoring post is supported by a competent, qualified person who is also authorised by the Licensed Water Sports Centre. |  |  |
| 7.3.9 | The Licensed Water Sports Centre has appropriate staff able to communicate matters of pricing policy and safety rules of the Maritime Recreational Means, adequately and effectively, to all users (e.g. with foreign persons). |  |  |
| 7.3.10 | The Licensed Water Sports Centre has set out defined positions and has means of effectively safeguarding the Maritime Recreational Means (e.g. closed circuit cameras, paid private security, etc.). |  |  |
| 7.3.11 | The Licensed Water Sports Centre has installed appropriate and specified buoys for the boundaries of the bathers' area, near the waterway and the operating area of its crafts. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.3.12 | Every user (irrespective of age) of a towed, motorised and non-motorised Maritime Recreational Means, signs a consent form for their participation in the activity. |  |  |
| 7.3.13 | The Licensed Water Sports Centre takes care to provide visual (or other suitable) information on the safety rules of the Maritime Recreational Means (as well as signals for the towed Maritime Recreational Means) for specific user groups (e.g. children, etc.) when using them. |  |  |
| 7.3.14 | The Licensed Water Sports Centre has a physician (e.g. workplace doctor) or access to a physician (appropriate medical specialist) for the provision of first aid, available during all hours of its operation. |  |  |
| 7.3.15 | The Licensed Water Sports Centre has access or takes care to provide easy approach for ambulances (or other rescue means) in the case the relative need arises for a user of Maritime Recreational Means. |  |  |
| 7.3.16 | The Licensed Water Sports Centre carries out regular checks (tests) for the effective communication between its members during full activity hours. |  |  |

| Number | CRITERION 3: Performance of Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.3.17 | The Licensed Water Sports Centre carries out a practice effective response to an emergency situation, on an annual basis, with the involvement of all its competent staff. |  |  |
| 7.3.18 | The Licensed Water Sports Centre informs the users of the towed Maritime Recreational Means about hand signals. |  |  |
| 7.3.19 | The company takes care to maintain the staff involved in the provision of Maritime Recreational means (in particular operators of the towing speedboats) in an optimal physical condition for the performance of their duties (e.g. work hours, appropriate nutrition, rest, etc.). |  |  |
| 7.3.20 | The temporary storage of spare fuel for the company's motorised equipment takes place in a controlled area and is inaccessible to users. |  |  |

| Number | CRITERION 4:Support Equipment for Maritime Recreational Means Services | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.4.1 | The Licensed Water Sports Centre documents the regular monitoring during its period of operation, with regard to the continuous sufficiency of available First Aid materials and means. |  |  |
| 7.4.2 | A CPR Face Shield is available. |  |  |
| 7.4.3 | The company takes care of the cleaning, but also the maintenance of the quality of the life jackets after each use, with the appropriate means to this end. |  |  |
| 7.4.4 | The Licensed Water Sports Centre has the necessary equipment for the effective observation of customers, users of Maritime Recreational Means (e.g. binoculars, etc.). |  |  |
| 7.4.5 | For the Maritime Recreational Means that are being used simultaneously by two or more users in close proximity (e.g. a sea sled), the company also provides, on a case-by-case basis, the suitable personal protection equipment, e.g. helmets, and encourages their use. |  |  |

| Number | CRITERION 5 :Support Facilities for Maritime Recreational Means | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.5.1 | The company provides its users with dressing rooms or showers for their proper preparation before and/or after using Maritime Recreational Means. |  |  |
| 7.5.2 | The company provides free access to clean toilets to accommodate the needs of users. |  |  |
| 7.5.3 | The company has a clean and tidy area for reception and waiting of users. |  |  |

| Number | CRITERION 6:Improving Services and Performance | Initial Certification Audit Data - Examples | Α |
| --- | --- | --- | --- |
| 7.6.1 | The Licensed Water Sports Centre proceeds in the evaluation and appraisal of the corrective actions in order to establish a relevant programme to avoid the recurrence of the same problems/failures. |  |  |
| 7.6.2 | The Licensed Water Sports Centre delegates responsibilities for the implementation of decisions for improving its services. |  |  |
| 7.6.3 | The Licensed Water Sports Centre provides its users with the means (e.g. website, mobile application, etc.) for pre-booking rental services of Maritime Recreational Means. |  |  |
| 7.6.4 | The Licensed Water Sports Centre provides its users with ancillary actions that may upgrade the service without affecting its level of safety (e.g. photographs of the users, etc.). |  |  |
| 7.6.5 | The company hangs, in a prominent position, the "Charter"/policy of the entitlements provided by the same and anticipated by the user during the use of Maritime Recreational Means. |  |  |

| "STATEMENT OF SATISFACTION OF OPTIONAL (BASIC) REQUIREMENTS OF WSCQ" |  |  |  |
| --- | --- | --- | --- |
| Basic Company Details |  |  |  |
| Company Name |  |  |  |
| Full Address (Street / number / Postal Code / City / Beach) |  |  |  |
| Lessor |  |  |  |
| Responsible by law |  |  |  |
| Date of checklist completion |  |  |  |
| Rental License for Maritime Recreational Means |  |  |  |
| Licensing Port Authority |  |  |  |
| Company description (number of employees, main activities, commercial activities, brief history) |  |  |  |
| Instructions for completion: Choose one of the automatic responses that appears by selecting the "Coverage of requirement by Licensed Water Sports Centre” cell. In the case of comments regarding the requirement, fill in the "Comments" cell |  |  |  |
| Number | CRITERION 1:Organisation and operation | COVERAGE OF REQUIREMENTS BY LICENSED WATER SPORTS CENTRE | COMMENTS |
| 7.1.1 | The Licensed Water Sports Centre has sufficient documentation and records which (a) allow the organised and (b) document its effective operation, in terms of quality of the services provided to its users/customers. |  |  |
| 7.1.2 | The Licensed Water Sports Centre manages its documents and records in such a way as to ensure their identification, updating them when necessary, to avoid unintended use of obsolete documents/records, and their current versions are available at the points of use. |  |  |

| 7.1.3 | The Licensed Water Sports Centre records, for all the (motorised and not) maritime recreational means it rents, their systematic monitoring/ evaluation of their quality. |  |  |
| --- | --- | --- | --- |
| 7.1.4 | The Licensed Water Sports Centre records all available life jackets, with reference to their size (children, adults, S, M, L, XL), as well as any comments about them. |  |  |
| 7.1.5 | The Licensed Water Sports Centre records all the other available equipment for the safety and protection of users with regard to the performance of the Maritime Recreational Means (e.g. helmets, communication devices, First Aid materials and means, etc.). |  |  |
|  | CRITERION 2:Staff and Responsibilities | COVERAGE OF REQUIREMENTS BY LICENSED WATER SPORTS CENTRE | COMMENTS |
| 7.2.1 | For the provision of Kite Surfing, the Licensed Water Sports Centre has an instructor whose competence is documented by a relevant certificate from an official specialised body. |  |  |
| 7.2.2 | The Licensed Water Sports Centre describes the tasks, duties and areas of responsibility of the staff. |  |  |
| 7.2.3 | The training/briefing of staff, on the company's policy and procedures on health and safety matters during the performance of the work, is documented. |  |  |
| 7.2.4 | The minimum qualifications required (e.g. work experience, skills, etc.) per role are determined. |  |  |
| 7.2.5 | Confirmation of the understanding of the duties and responsibilities of staff is documented. |  |  |
| 7.2.6 | The regular assessment of the staff's response with regard to the quality of their work is documented and conducted by a person with relevant professional competence. |  |  |
| 7.2.7 | The Licensed Water Sports Centre documents the availability of an adequate and sufficient number of staff for the provision of its services throughout the duration of its operation. |  |  |
|  | CRITERION 3: Performance of Maritime Recreational Means Services | COVERAGE OF REQUIREMENTS BY LICENSED WATER SPORTS CENTRE | COMMENTS |

| 7.3.1 | While the Maritime Recreational Means are being towed by a speedboat, apart from the qualified driver (according to par. 6.2.9) there is also a second person on board who is a member of the staff of the company with relevant training. |  |  |
| --- | --- | --- | --- |
| 7.3.2 | The company maintains a record of the data of users of motorised equipment e.g. jet-ski for which documented knowledge of how to handle them is required. |  |  |
| 7.3.3 | The Licensed Water Sports Centre has identified a demarcated sea area/arena within which the Maritime Recreational Means are allowed to operate (particularly for motorised Maritime Recreational Means, e.g. Jet ski, etc.). |  |  |
| 7.3.4 | The correct functioning of the boat used as a rescue boat is documented. |  |  |
| 7.3.5 | The company has a safe approach procedure for the collection of users in the event of their fall from the Maritime Recreational Means. |  |  |
| 7.3.6 | The lessor or the employee is the holder of a CPR certificate. |  |  |
| 7.3.7 | The Licensed Water Sports Centre has defined and documents (e.g. sketch of the post) the manned point for monitoring the Maritime Recreational Means. |  |  |
| 7.3.8 | During rental of maritime recreational means, the monitoring post is supported by a competent, qualified person who is also authorised by the Licensed Water Sports Centre. |  |  |
| 7.3.9 | The Licensed Water Sports Centre has appropriate staff able to communicate matters of pricing policy and safety rules of the Maritime Recreational Means, adequately and effectively, to all users (e.g. with foreign persons). |  |  |
| 7.3.10 | The Licensed Water Sports Centre has set out defined positions and has means of effectively safeguarding the Maritime Recreational Means (e.g. closed circuit cameras, paid private security, etc.). |  |  |
| 7.3.11 | The Licensed Water Sports Centre has installed appropriate and specified buoys for the boundaries of the bathers' area, near the waterway and the operating area of its crafts. |  |  |
| 7.3.12 | Every user of a towed, motorised and non-motorised Maritime Recreational Means, signs a consent form for their participation in the activity. |  |  |

| 7.3.13 | The Licensed Water Sports Centre takes care to provide visual (or other suitable) information on the safety rules of the Maritime Recreational Means (as well as signals for the towed Maritime Recreational Means) for specific user groups (e.g. children , etc.) when using them. |  |  |
| --- | --- | --- | --- |
| 7.3.14 | The Licensed Water Sports Centre has a physician (e.g. workplace doctor) or access to a physician (appropriate medical specialist) for the provision of first aid, available during all hours of its operation. |  |  |
| 7.3.15 | The Licensed Water Sports Centre has access or takes care to provide easy approach for ambulances (or other rescue means) in the case the relative need arises for a user of Maritime Recreational Means. |  |  |
| 7.3.16 | The Licensed Water Sports Centre carries out regular checks (tests) for the effective communication between its members during full activity hours. |  |  |
| 7.3.17 | The Licensed Water Sports Centre carries out a practice effective response to an emergency situation, on an annual basis, with the involvement of all its competent staff. |  |  |
| 7.3.18 | The Licensed Water Sports Centre informs the users of the towed Maritime Recreational Means about hand signals. |  |  |
| 7.3.19 | The company takes care to maintain the staff involved in the provision of Maritime Recreational means (in particular operators of the towing speedboats) in an optimal physical condition for the performance of their duties (e.g. work hours, appropriate nutrition, rest, etc.). |  |  |
| 7.3.20 | The temporary storage of spare fuel for the company's motorised equipment takes place in a controlled area and is inaccessible to the users. |  |  |
|  | CRITERION 4:Support Equipment for Maritime Recreational Means Services | COVERAGE OF REQUIREMENTS BY LICENSED WATER SPORTS CENTRE | COMMENTS |
| 7.4.1 | The Licensed Water Sports Centre documents the regular monitoring during its period of operation, with regard to the continuous sufficiency of available First Aid materials and means. |  |  |
| 7.4.2 | A CPR Face Shield is available. |  |  |

| 7.4.3 | The company takes care of the cleaning, but also the maintenance of the quality of the life jackets, after each use, with the appropriate means to this end. |  |  |
| --- | --- | --- | --- |
| 7.4.4 | The Licensed Water Sports Centre has the necessary equipment for the effective observation of customers, users of Maritime Recreational Means (e.g. binoculars, etc.). |  |  |
| 7.4.5 | For the Maritime Recreational Means that are being used simultaneously by two or more users in a close proximity (e.g. a sea sled), the company also provides, on a case-by-case basis, the suitable personal protection, e.g. helmets, and encourages their use. |  |  |
|  | CRITERION 5:Support Facilities for Maritime Recreational Means | COVERAGE OF REQUIREMENTS BY LICENSED WATER SPORTS CENTRE | COMMENTS |
| 7.5.1 | The company provides its users with dressing rooms or showers for their proper preparation before and/or after using the Maritime Recreational Means. |  |  |
| 7.5.2 | The company provides free access to clean toilets to accommodate the needs of users. |  |  |
| 7.5.3 | The company has a clean and tidy area for reception and waiting of users. |  |  |
|  | CRITERION 6:Improving Services and Performance | COVERAGE OF REQUIREMENTS BY LICENSED WATER SPORTS CENTRE | COMMENTS |
| 7.6.1 | The Licensed Water Sports Centre proceeds in the evaluation and appraisal of the corrective actions in order to establish a relevant programme to avoid the recurrence of the same problems/failures. |  |  |
| 7.6.2 | The Licensed Water Sports Centre delegates responsibilities for the implementation of decisions for improving its services. |  |  |
| 7.6.3 | The Licensed Water Sports Centre provides its users with the means (e.g. website, mobile application, etc.) for pre-booking rental services of Maritime Recreational Means. |  |  |
| 7.6.4 | The Licensed Water Sports Centre provides its users with ancillary actions that may upgrade the service without affecting its level of safety (e.g. photographs of the users, etc.). |  |  |

| 7.6.5 | The company hangs, in a prominent position, the "Charter"/policy of the entitlements provided by the same and anticipated by the user during the use of Maritime Recreational Means. |  |  |
| --- | --- | --- | --- |

  - | Annex 3: Application for the inclusion under the Quality Label

Ü

V

A

U

S

T

R

I

A

APPLICATION FOR THE INCLUSION UNDER THE 'WATER SPORTS CERTIFIED QUALITY' QUALITY LABEL

| INTERESTED COMPANY’S INFORMATION |  |  |
| --- | --- | --- |
| COMPANY NAME |  |  |
| TRADE NAME |  |  |
| REGISTERED OFFICES | POSTAL CODE- CITY |  |
|  | FAX |  |
|  | URL |  |
| TAX REGISTRATION NO. | TAX OFFICE |  |
| LOCATION WHERE INSPECTION WILL TAKE PLACE: |  |  |
| AUTHORISED REPRESENTATIVE OF THE COMPANY: |  |  |
| CONSULTANT |  |  |
| SCOPE OF THE CERTIFICATION (REFER TO THE RENTED MARITIME RECREATIONAL MEANS) |  |  |
| MOTORISED MEANS | NON-MOTORISED MEANS |  |
| EMPLOYEES - PERMANENT PERSONNEL | SHIFTS |  |
| SEASONAL PERSONNEL | NUMBER OF STAFF PER SHIFT |  |
| DETAILS FOR THE AUDIT AND CERTIFICATION |  |  |
| RENTAL LICENSE MARITIME RECREATIONAL MEANS |  |  |
| LICENSING PORT AUTHORITY |  |  |
| PREFERRED INSPECTION PERIOD (MONTH / YEAR): |  |  |
| OTHER INFORMATION COMMENTS |  |  |
| In addition, I responsibly declare that the company has in place all the necessary legislative documentation pertaining to its operation. |  |  |
| DATE |  | STAMP/SIGNATURE |
| Please send to TÜV AUSTRIA HELLAS on FAX No.: 2 1 0 5 2 0 3 9 9 0 or at certification@tuvaustriahellas.gr For any further information or clarifications please do not hesitate to contact us on Tel. No.: 2 1 0 5 2 2 0 9 2 0 |  |  |
| Date | APPLICATION REVIEW (to be filled in by the Certification Body) | Signature |

T

ATHENS429 MesogeionAvenue153 43 Agia Paraskevi, T:210 5220920, F:210 5203990, E: info@tuvaustriahellas.gr

THESSALONIKI:8 Chalkis Street, Pylaia, 10th km Thessaloniki-Mounadion - Postal Code 570 01, PO Box 4207, T:2310 941100, F:2310 941105, E: thessaloniki@tuvaustriahellas.gr

HERAKLION:Kifisias6 Andrea Papandreou Avenue - 713 05, T:2810 244150, F:2810 244551, E: iraklio@tuvaustriahellas.gr

MYTILENE:57 Theokritou, 811 00, Mytilene, T:22510 40504, F:22510 40502, E: mitilini@tuvaustriahellas.gr ABROAD: CYPRUS, ALBANIA, TURKEY, EGYPT, JORDAN, QATAR, PAKISTAN

www.tuvaustriahellas.gr

  - |Annex 4:  Certificate

| CERTIFICATE |  |
| --- | --- |
| For the requirements of the Water Sports Certified Quality Protocol |  |
| It has been documented and confirmed by means of the Procedures of TÜV AUSTRIA that |  |
| Full Name of Licensed Water Sports Centre Address / Installation Site of Licensed Water Sports Centre |  |
| Applies the requirements for the quality of support and services provided, in accordance with the above Protocol for the following scope: Rental XXXX XXXXXXXXXXXXXXX |  |
| Level of Compliance: | PLATINUM |
|  | GOLD |
|  | SILVER |
|  | BRONZE |

### HEADQUARTERS:

#### 429 MesogeionAvenue, 15343

Agia Paraskevi

Τ: +30 210 5220920

F: +30 210 5203990

E: info@tuvaustriahellas.gr

BRANCH OF NORTHERN GREECE:

8 Chalkis Street, 10th km National Road

Thessaloniki - Moudanion 570 01, Pylaia Thessaloniki

PO Box: 4207

#### Τ: +30 2310 941100

F: +30 2310 941105

E: thessaloniki@tuvaustriahellas.gr

BRANCH OF CRETE:

Renewal Audit to be conducted by

6 Andrea Papandreou Avenue

F: +30 2810 244551

E: iraklio@tuvaustriahellas.gr

AEGEAN BRANCH:

57 Theokritou Street

81100, Mitilini

Τ: +30 22510 40504-5

F: +30 22510 40502

E: mitilini@tuvaustriahellas.gr

Water Sports Certified Quality Protocol, Version 2 - January 2018